## AERODROME LOCATION INDICATOR AND NAME

EADD — DONLON/International

## AERODROME GEOGRAPHICAL AND ADMINISTRATIVE DATA

| 1 | ARP coordinates and site at AD | 522318N 0315658W 258°/1075 M from THR 09L |
| --- | --- | --- |
| 2 | Direction and distance from (city) | 045°, 9 KM from Donlon |
| 3 | Elevation/Reference temperature | 30 M/21°C |
| 4 | Geoid undulation at AD ELEV PSN | 12 M |
| 5 | Magnetic (MAG) variation (VAR)/Annual change | 3°W (1990)/0.03° decreasing |
| 6 | Name of aerodrome operator, address, telephone, telefax numbers, e-mail address, AFS address and, if available, website address | Civil Aviation Administration Donlon Airport Donlon 4 W Tel: 01238282 Telefax: 01238292 E-mail: contact@ibosoft.net.tr AFS: EADDYDYX Website: www.ibosoft.net.tr |
| 7 | Types of traffic permitted (IFR/VFR) | IFR/VFR |
| 8 | Remarks | NIL |

## OPERATIONAL HOURS

| 1 | Aerodrome Operator | MON–FRI: 0600–2000 (0500–1900) SAT, SUN + HOL: 0700–2000 (0600–1900) |
| --- | --- | --- |
| 2 | Customs and immigration | MON–FRI: 0900–1800 (0800–1700) SAT, SUN + HOL: 1000–1700 (0900–1600) |
| 3 | Health and sanitation | Available within AD hours. 2 HR PN to AD required |
| 4 | Aeronautical information service (AIS) briefing office | As AD administration. |
| 5 | ATS Reporting Office (ARO) | As AD administration. |
| 6 | MET Briefing Office | As AD administration. |
| 7 | ATS | As AD administration. |
| 8 | Fuelling | As AD administration. |
| 9 | Handling | As AD administration. |
| 10 | Security | As AD administration. |
| 11 | De-icing | As AD administration. |
| 12 | Remarks | Outside these hours, services are available O/R. Request to be submitted to the AD not later than 1500 (1400) UTC. |

## HANDLING SERVICES AND FACILITIES

| 1 | Cargo-handling facilities | Trucks 1.5–3.5 tonnes. Up to 10 tonnes handling possible. |
| --- | --- | --- |
| 2 | Fuel/oil types | Jet A1, AVTUR, AVGAS 100 LL, oil, all types normally available. |
| 3 | Fuelling facilities/capacity | 1 truck 45 000 litres, 50 litres/sec. |
| 4 | De-icing facilities | Available. See AD chart for location |
| 5 | Hangar space for visiting aircraft | Limited, by prior arrangement only. |
| 6 | Repair facilities for visiting aircraft | Available for aircraft up to 5 700 KG. Major repairs by arrangement. |
| 7 | Remarks | Handling services available within AD HR or by arrangement with the AD. |

## PASSENGER FACILITIES

| 1 | Hotels | Near the AD and in the city. |
| --- | --- | --- |
| 2 | Restaurants | At AD and in the city. |
| 3 | Transportation | Buses, taxis and car hire from the AD. Trains to and from the city. |
| 4 | Medical facilities | First aid at AD. Hospitals in the city. |
| 5 | Bank and Post Office | At AD. Open within AD HR. |
| 6 | Tourist Office | Office in the city. Tel: Donlon 0123 4863559 Telefax: 0123 4863569 |
| 7 | Remarks | AD website: www.donlonairport.com/passengers |

## RESCUE AND FIRE-FIGHTING SERVICES

| 1 | AD category for fire-fighting | Within AD HR: CAT 7 |
| --- | --- | --- |
| 2 | Rescue equipment | Yes, 2 boats of 40 persons |
| 3 | Capability for removal of disabled aircraft | Lifting bags and hydraulic jacks available |
| 4 | Remarks | Outside AD HR, fire-fighting service to be requested. Request to be submitted not later than 1500 (1400) UTC. |

## SEASONAL AVAILABILITY — CLEARING

| 1 | Types of clearing equipment | 1 snow blower; 2 snow ploughs; 2 scrapers; 1 sand spreader |
| --- | --- | --- |
| 2 | Clearance priorities | 1. Runway (RWY) 09L/27R and associated taxiway (TWY) to apron 2. RWY 09R/27L and TWY to apron 3. Other TWY and aircraft (ACFT) stands |
| 3 | Remarks | Information on snow clearance published from November– April in NOTAM (SNOWTAM). See also the snow plan in section AD 1.2.2 |

## APRONS, TAXIWAYS AND CHECK LOCATIONS/POSITIONS DATA

| 1 | Apron designation, surface and strength | Apron A, Asphalt, PCN 80/R/B/W/T Apron B, Concrete, PCN 80/R/B/W/T |
| --- | --- | --- |
| 2 | Taxiway designation, width, surface and strength | TWY A, 23 M, Asphalt, PCN 80/R/B/W/T TWY B, 20 M, Concrete, PCN 80/R/B/W/T |
| 3 | Altimeter checkpoint location and elevation | Location: At Apron Elevation: 28 M |
| 4 | VHF omnidirectional radio range (VOR) checkpoints | VOR: See AD chart |
| 5 | INS checkpoints | INS: See AD chart |
| 6 | Remarks | NIL |

## SURFACE MOVEMENT GUIDANCE AND CONTROL SYSTEM AND MARKINGS

| 1 | Use of aircraft stand ID signs, TWY guide lines and visual docking/parking guidance system of aircraft stands | Taxiing guidance signs at all intersections with TWY and RWY and at all holding positions. Guide lines at apron. Nose-in guidance at aircraft stands. |
| --- | --- | --- |
| 2 | RWY and TWY markings and LGT | RWY: Designation, threshold (THR), touch-down zone (TDZ), centre line, edge runway end as appropriate, marked and lighted. TWY: Centre line, holding positions at all TWY/RWY intersections, marked and lighted. |
| 3 | Stop bars | Stop bars where appropriate |
| 4 | Other runway protection measures | NIL |
| 5 | Remarks | See also page ........... (specify) for taxiing to and from stands. |

## AERODROME OBSTACLES

| In Area 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OBST ID/ Designation | OBST type | OBST position | ELEV/HGT | Markings/ Type, colour, lighting (LGT) | Remarks |
| a | b | c | d | e | f |
| EADDOB001 | Antenna | 522142.17N 0320215.24W | 93/60 M | MARKED/FLS W | Obstacle data sets are available (see GEN 3.1.6) |
| EADDOB002 | Power line | 522151.82N 0315845.12W | 65/15 M | MARKED |  |
| EADDOB003 | Tower | 522203.36N 0315457.22W | 40/12 M | LGTD |  |
| EADDOB004 | Mobile OBST | 522243.85N 0315455.58W | 28/3 M | NIL |  |

| In Area 3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OBST ID/ Designation | OBST type | OBST position | ELEV/HGT | Markings/ Type, colour, lighting (LGT) | Remarks |
| a | b | c | d | e | f |
| EADDOB005 | Terminal building | 522124.86N 0315452.18W | 31.5/15 M | MARKED/HI R | Obstacle data sets are Available (see GEN 3.1.6) |
| EADDOB006 | Hangar | 522115.34N 0315532.17W | 55/20 M | LGTD |  |
| EADDOB007 | Antenna | 522138.15N 0315425.48W | 37/4 M | LGTD |  |

## METEOROLOGICAL INFORMATION PROVIDED

| 1 | Associated MET office | DONLON |
| --- | --- | --- |
| 2 | Hours of service MET office outside hours | H24 |
| 3 | Office responsible for terminal aerodrome forecast (TAF) preparation Periods of validity | DONLON 9,18 HR |
| 4 | Trend forecast Interval of issuance | TREND 1 HR |
| 5 | Briefing/consultation provided | Personal consultation, closed circuit television |
| 6 | Flight documentation Language(s) used | Charts, abbreviated plain language text English |
| 7 | Charts and other information available for briefing or consultation | S, U85, U70, U50, U30, U20, P85, P70, P50, P40, P30 P20, SWH, SWM, T |
| 8 | Supplementary equipment available for providing information | Telefax; self-briefing terminal; weather radar; satellite receiver |
| 9 | ATS units provided with information | Donlon TWR; Donlon APP |
| 10 | Additional information (limitation of service, etc.) | Nil |

## RUNWAY PHYSICAL CHARACTERISTICS

| Designations RWY NR | TRUE BRG | Dimensions of RWY (M) | Strength of the pavement classification rating (PCR) and surface of RWY and SWY | THR coordinates RWY end coordinates THR geoid undulation | THR elevation and highest elevation of TDZ of precision APP RWY |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 09L | 085.23° | 2 800 × 45 | 760/R/B/W/T Concrete | 522232.15N 0315751.35W GUND 11.5 M | THR 30 M/99 FT |
| 27R | 265.23° | 2 800 × 45 | 760/R/B/W/T Concrete | 522241.48N 0315518.65W GUND 11.5 M | THR 16.5 M/53 FT |
| 09R | 085.29° | 2 600 × 45 | 550/F/A/Y/U Asphalt/ Concrete | 522155.82N 0315754.03W GUND 11.5 M | TDZ 20.5 M/66 FT |
| 27L | 265.29° | 2 600 × 45 | 550/F/A/Y/U Asphalt/ Concrete | 522205.71N 0315532.14N GUND 11.5 M | THR 20 M/66 FT |
| Designations RWY NR | Slope of RWY-SWY | SWY dimensions (M) | Clearway (CWY) dimensions (M) | Strip dimensions (M) | Dimensions of runway end safety areas |
| 1 | 7 | 8 | 9 | 10 | 11 |
| 09L | 0.5% | NIL | NIL | 2 920 × 300 | 180 x 90 |
| 27R | 0.5% | NIL | NIL | 2 920 × 300 | 200 x 90 |
| 09R | +1%/–1% (1600 M) (1000 M) | 200 × 45 | NIL | 2 920 × 300 | 240 x 90 |
| 27L | +1%/–1% (1000 M) (1600 M) | 200 × 45 | 400 × 150 | 2 920 × 150 | 160 x 90 |

| Designations RWY NR | Location and description of engineering material arresting system (EMAS) | OFZ | Remarks |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 12 | 13 | 14 |  |  |
| 09L | NIL | NIL | NIL |  |  |
| 27R | NIL | NIL | NIL |  |  |
| 09R | NIL | NIL | NIL |  |  |
| 27L | End of RWY 27L EMAS with a length of 160 m and a width of 45 m at the end of. | NIL | NIL |  |  |

## DECLARED DISTANCES

| RWY designator | TORA (M) | TODA (M) | ASDA (M) | LDA (M) | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 09L | 2 800 | 2 800 | 2 800 | 2 800 | NIL |
| 27R | 2 800 | 2 800 | 2 800 | 2 500 | DTHR 1 300 M |
| 09R | 2 600 | 2 600 | 2 600 | 2 600 | NIL |
| 27L | 2 600 | 3 000 | 2 800 | 2 600 | NIL |

## APPROACH AND RUNWAY LIGHTING

| RWY designator | APCH LGT type LEN INTST | THR LGT colour WBAR | VASIS (MEHT) PAPI | TDZ, LGT LEN | RWY Centre Line LGT Length, spacing, colour, INTST | RWY edge LGT LEN, spacing colour INTST | RWY End LGT colour WBAR | SWY LGT LEN (M) colour | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| 09L | SIAL 600 M LIM | Green – | PAPI Left/3° (30 FT) | NIL | 2 800 M, 30 M White, LIH | 2 800 M, 50 M White, LIH | Red – | NIL | NIL |
| 27R | CAT II 900 M LIH | Green – | PAPI Left/3° (69 FT) | 900 M | 2 800 M, 7.5 M White; FM 1900 M–2500 M Red/White; FM 2 500 M Red; LIH | 2 800 M, 50 M White, LIH | Red – | NIL | NIL |
| 09R | NIL | Green – | PAPI 3.75° (28 FT) | NIL | NIL | 2 600 M, 50 M White, LIM | Red – | 200 M Red | NIL |
| 27L | NIL | Green – | T-VASIS 2.75° (40 FT) | NIL | NIL | 2 600 M, 50 M White, LIM | Red – | 200 M Red | NIL |

## OTHER LIGHTING, SECONDARY POWER SUPPLY

| 1 | ABN/IBN location, characteristics and hours of operation | ABN: At Tower Building, FLG W EV 2 SEC/IBN: NIL H24 |
| --- | --- | --- |
| 2 | LDI location and LGT Anemometer location and LGT | LDI: 800 M W of ARP, lighted Anemometer: 300 M from THR 09L, not lighted |
| 3 | TWY edge lights, centre line lights and stop bars (if any) | Edge: All TWY Centre line: TWY A, B, C, D, E Stop bars: All TWY/RWY intersections |
| 4 | Secondary power supply/switch-over time | Secondary power supply to all lighting at AD. Switch-over time: 1 SEC |
| 5 | Remarks | NIL |

## HELICOPTER LANDING AREA

| 1 | Coordinates touchdown and lift-off (TLOF) or THR of final approach and take-off (FATO) Geoid undulation | 522226.98N 0315636.61W 12.5 M/41.5 FT |
| --- | --- | --- |
| 2 | TLOF and/or FATO elevation M/FT | 33 M/109 FT |
| 3 | TLOF and FATO area dimensions, surface, strength, marking | Rectangle 30 x 30 M, asphalt, 10 tonnes, white edges and white letter H |
| 4 | True BRG of FATO | 123.25/303.25° Direction of TKOF zones: 124° GEO 304° GEO |
| 5 | Declared distance available | NIL |
| 6 | APP and FATO lighting | FATO area edge, air TWY to apron |
| 7 | Remarks | NIL |

## ATS AIRSPACE

| 1 | Designation and lateral limits | DONLON CTR A circle, radius 35 KM centred at 522318N 0315658W (ARP) |
| --- | --- | --- |
| 2 | Vertical limits | Surface (SFC) to 3 000 FT MSL |
| 3 | Airspace classification | D |
| 4 | ATS unit call sign Language(s) | Donlon Tower English |
| 5 | Transition altitude | 3 500 FT MSL |
| 6 | Hours of applicability (or activation) | MON-FRI 0530-2000 (0430-1900) SAT, SUN + HOL: 0700-2000 (0600-1900) |
| 7 | Remarks | NIL |

## ATS COMMUNICATION FACILITIES

| Service designation | Call sign | Frequency | Hours of operation | Remarks |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 |
| APP | Donlon Approach | 119.100 121.500 | H24 H24 | Primary frequency Emergency frequency |
| TWR | Donlon Tower | 118.100 117.900 119.900 | As AD HO HO | Primary frequency Military aircraft |
| SRE | Donlon Director | 123.700 118.100 | 0700–2100 (0600–2000) O/R | Primary frequency |
| PAR | Donlon Precision | 119.900 | O/R 0700–2100 (0600–2000) | For RWY 27R. Primary frequency |
| ATIS (ARR) | Donlon Arrival Information | 122.750 | 0600–2200 (0500–2100) |  |
| ATIS (DEP) | Donlon Departure Information | 122.850 | 0600–2200 (0500–2100) |  |
| ATIS (INF) | Donlon Information | 122.750 | 2200–0600 (2100–0500) |  |

## RADIO NAVIGATION AND LANDING AIDS

| Type of aid, MAG VAR, Type of supported OPS for ILS/MLS/GLS, Basic GNSS/SBAS, Classification for ILS, Facility classification and APP facility designator(s) for GBAS, Station declination for VOR/ILS/MLS | ID | FREQ, CH NR, Service provider, RPI | Hours of OPS | Position of transmitting antenna coordinates | ELEV of DME transmitting antenna | Service volume radius from the GBAS reference point | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| VOR/DME (3°W/1990) | BOR | 116.900 MHz CH 116X | H24 | 522206.2N 0322230.8W | 60 M |  |  |
| VOR/DME (3°W/1990) | CAA | 114.300 MHz CH 90X | H24 | 522254.4N 0314436.1W | 30 M |  |  |
| VOR/DME (3°W/1990) | KAV | 115.000 MHz CH 97X | H24 | 523218.3N 0315512.6W | 30 M |  |  |
| L | KL | 411 KHz | H24 | 522301.2N 0315102.3W |  |  | 087° MAG/5.7 KM to RWY 27R. Coverage 25 NM |
| LLZ 27R (3°W/1990) ILS CAT II (3°W, 267° GEO) | IOXS | 109.100 MHz CH 28X | H24 | 522232.1N 0315754.8W |  |  |  |
| GP 27R |  | 331.400 MHz | H24 | 522242.4N 0315536.4W |  |  | 3°, RDH 51 FT |
| DME 27R | IOXS | 109.100 MHz CH 28X | H24 | 522242.4N 0315536.4W |  |  | Coverage 25 NM |
| MM 27R | Dots/ Dashes | 75 KHz | H24 | 522246.8N 0315422.8W |  |  | 087° MAG/1.1 KM to RWY 27R. Minor axis 267° GEO |
| OM 27R | Dashes | 75 KHz | H24 | 522301.2N 0315102.3W |  |  | 087° MAG/5.7 KM to RWY 27R. Minor axis 267° GEO |
| GPS NPA |  | 1575.42 MHz | H24 | N/A |  |  |  |
| WAAS LPV |  | 1575.42 MHz | H24 | N/A |  |  |  |
| GBAS CAT I | ERWN | 117.900 MHz 126X | H24 | 522244.4N 0315536.4W |  | 20 NM |  |

## LOCAL AERODROME REGULATIONS

### Heading 3

#### Heading 4

Lorem ipsum dolor sit amet, consectetur adipiscing elit, sed do eiusmod tempor incididunt ut labore et dolore magna aliqua. Ut enim ad minim veniam, quis nostrud exercitation ullamco laboris nisi ut aliquip ex ea commodo consequat.

#### Heading 4

Lorem ipsum dolor sit amet, consectetur adipiscing elit, sed do eiusmod tempor incididunt ut labore et dolore magna aliqua. Ut enim ad minim veniam, quis nostrud exercitation ullamco laboris nisi ut aliquip ex ea commodo consequat.

### Heading 3

Lorem ipsum dolor sit amet, consectetur adipiscing elit, sed do eiusmod tempor incididunt ut labore et dolore magna aliqua. Ut enim ad minim veniam, quis nostrud exercitation ullamco laboris nisi ut aliquip ex ea commodo consequat.

## NOISE ABATEMENT PROCEDURES

### Heading 3

#### Heading 4

Lorem ipsum dolor sit amet, consectetur adipiscing elit, sed do eiusmod tempor incididunt ut labore et dolore magna aliqua. Ut enim ad minim veniam, quis nostrud exercitation ullamco laboris nisi ut aliquip ex ea commodo consequat.

#### Heading 4

Lorem ipsum dolor sit amet, consectetur adipiscing elit, sed do eiusmod tempor incididunt ut labore et dolore magna aliqua. Ut enim ad minim veniam, quis nostrud exercitation ullamco laboris nisi ut aliquip ex ea commodo consequat.

### Heading 3

Lorem ipsum dolor sit amet, consectetur adipiscing elit, sed do eiusmod tempor incididunt ut labore et dolore magna aliqua. Ut enim ad minim veniam, quis nostrud exercitation ullamco laboris nisi ut aliquip ex ea commodo consequat.

## FLIGHT PROCEDURES

### Heading 3

#### Heading 4

Lorem ipsum dolor sit amet, consectetur adipiscing elit, sed do eiusmod tempor incididunt ut labore et dolore magna aliqua. Ut enim ad minim veniam, quis nostrud exercitation ullamco laboris nisi ut aliquip ex ea commodo consequat.

#### Heading 4

Lorem ipsum dolor sit amet, consectetur adipiscing elit, sed do eiusmod tempor incididunt ut labore et dolore magna aliqua. Ut enim ad minim veniam, quis nostrud exercitation ullamco laboris nisi ut aliquip ex ea commodo consequat.

### Heading 3

Lorem ipsum dolor sit amet, consectetur adipiscing elit, sed do eiusmod tempor incididunt ut labore et dolore magna aliqua. Ut enim ad minim veniam, quis nostrud exercitation ullamco laboris nisi ut aliquip ex ea commodo consequat.

## ADDITIONAL INFORMATION

### Heading 3

#### Heading 4

Lorem ipsum dolor sit amet, consectetur adipiscing elit, sed do eiusmod tempor incididunt ut labore et dolore magna aliqua. Ut enim ad minim veniam, quis nostrud exercitation ullamco laboris nisi ut aliquip ex ea commodo consequat.

#### Heading 4

Lorem ipsum dolor sit amet, consectetur adipiscing elit, sed do eiusmod tempor incididunt ut labore et dolore magna aliqua. Ut enim ad minim veniam, quis nostrud exercitation ullamco laboris nisi ut aliquip ex ea commodo consequat.

### Heading 3

Lorem ipsum dolor sit amet, consectetur adipiscing elit, sed do eiusmod tempor incididunt ut labore et dolore magna aliqua. Ut enim ad minim veniam, quis nostrud exercitation ullamco laboris nisi ut aliquip ex ea commodo consequat.

## CHARTS RELATED TO AN AERODROME

Aerodrome/Heliport Chart — ICAO		AD2 EADD ADC

Aircraft Parking/Docking Chart — ICAO		AD2 EADD PRKG 1

Aerodrome Ground Movement Chart — ICAO		AD2 EADD GMC 1

Aerodrome Obstacle Chart — ICAO Type A	RWY 27R	AD2 EADD AOC 1

Aerodrome Obstacle Chart — ICAO Type B		AD2 EADD AOC 2

Aerodrome Terrain and Obstacle Chart (Electronic)

Precision Approach Terrain Chart — ICAO	RWY 27R	AD2 EADD PATC 1

Area Chart — ICAO (departure and transit routes)		AD2 EADD ARC 1

Standard Departure Chart — Instrument — ICAO	RWY 27R	AD2 EADD SID 1

Area Chart — ICAO (arrival and transit routes)		AD2 EADD ARC 2

Standard Arrival Chart — Instrument — ICAO	RWY 09L/27R	AD2 EADD STAR 1

ATC Surveillance Minimum Altitude Chart — ICAO		AD2 EADD ATCSMAC 1

Instrument Approach Chart — ICAO	ILS RWY 27R	AD2 EADD IAC 1

Instrument Approach Chart — ICAO	GLS RWY 27L	AD2 EADD IAC 2

Instrument Approach Chart — ICAO	RNP Z RWY 27L	AD2 EADD IAC 3

Visual Approach Chart — ICAO		AD2 EADD VAC 1

Bird concentrations in the vicinity of the aerodrome.		AD2 EADD Bird

## VISUAL SEGMENT SURFACE (VSS) PENETRATION

Lorem ipsum dolor sit amet, consectetur adipiscing elit, sed do eiusmod tempor incididunt ut labore et dolore magna aliqua. Ut enim ad minim veniam, quis nostrud exercitation ullamco laboris nisi ut aliquip ex ea commodo consequat.