## AERODROME LOCATION INDICATOR AND NAME

ZZZZ — BOLU

## AERODROME GEOGRAPHICAL AND ADMINISTRATIVE DATA

| 1 | ARP coordinates and site at AD | 404455N 0313906E 64.6° T/948 M from THR 06 |
| --- | --- | --- |
| 2 | Direction and distance from (city) | 057°, 2 NM from Bolu |
| 3 | Elevation/Reference temperature | 2441 FT/10°C |
| 4 | Geoid undulation at AD ELEV PSN | -124 FT |
| 5 | Magnetic (MAG) variation (VAR)/Annual change | 6.23°E* (2025)/Unknown |
| 6 | Name of aerodrome operator, address, telephone, telefax numbers, e-mail address, AFS address and, if available, website address | Ibosoft Kuruçay, D100, Merkez/Bolu, 14300 Tel: NIL Telefax: NIL E-mail: contact@ibosoft.net.tr AFS: NIL Website: ais.ibosoft.net.tr |
| 7 | Types of traffic permitted (IFR/VFR) | IFR/VFR |
| 8 | Remarks | 1. Usage: Civil/Military 2. GEN 1.3 – Type of traffic permitted to use the aerodrome/heliport: National (NTL) / IFR-VFR / Non-scheduled (N) - General Aviation (G) - Military (M) 3. No AFS/AFTN connection present. 4. AIXM DataType – CodeAirportHeliportBaseType = AD 5. *MAG VAR is not surveyed. The value is based on the World Magnetic Model 2025 (WMM 2025). |

## OPERATIONAL HOURS

| 1 | Aerodrome Operator | H24 |
| --- | --- | --- |
| 2 | Customs and immigration | NIL |
| 3 | Health and sanitation | H24 |
| 4 | Aeronautical information service (AIS) briefing office | NIL |
| 5 | ATS Reporting Office (ARO) | NIL |
| 6 | MET Briefing Office | NIL |
| 7 | ATS | O/R |
| 8 | Fuelling | Refer to AD administrator for further information. |
| 9 | Handling | NIL |
| 10 | Security | NIL |
| 11 | De-icing | NIL |
| 12 | Remarks | Refer to AD administrator for further information. AIP and AIS information for Bolu AD is provided by Ibosoft via Ibosoft AIS. Website: ais.ibosoft.net.tr |

## HANDLING SERVICES AND FACILITIES

| 1 | Cargo-handling facilities | Refer to AD administrator. |
| --- | --- | --- |
| 2 | Fuel/oil types | Diesel, gasoline, LPG from nearby gas stations. |
| 3 | Fuelling facilities/capacity | NIL |
| 4 | De-icing facilities | NIL |
| 5 | Hangar space for visiting aircraft | NIL |
| 6 | Repair facilities for visiting aircraft | NIL |
| 7 | Remarks | Refer to AD administrator for further information. |

## PASSENGER FACILITIES

| 1 | Hotels | In the city. |
| --- | --- | --- |
| 2 | Restaurants | At AD and in the city. |
| 3 | Transportation | Buses, taxis. |
| 4 | Medical facilities | First aid at AD. Hospitals in the city. |
| 5 | Bank and Post Office | In the city. |
| 6 | Tourist Office | In the city. |
| 7 | Remarks | AIS website: ais.ibosoft.net.tr |

## RESCUE AND FIRE-FIGHTING SERVICES

| 1 | AD category for fire-fighting | NIL |
| --- | --- | --- |
| 2 | Rescue equipment | NIL |
| 3 | Capability for removal of disabled aircraft | NIL |
| 4 | Remarks | NIL |

## SEASONAL AVAILABILITY — CLEARING

| 1 | Types of clearing equipment | NIL |
| --- | --- | --- |
| 2 | Clearance priorities | NIL |
| 3 | Remarks | NIL |

## APRONS, TAXIWAYS AND CHECK LOCATIONS/POSITIONS DATA

| 1 | Apron designation, surface and strength | Apron 1, Asphalt, NIL |
| --- | --- | --- |
| 2 | Taxiway designation, width, surface and strength | TWY A, 10 M, Asphalt, NIL |
| 3 | Altimeter checkpoint location and elevation | Location: At apron Elevation: 2435 FT |
| 4 | VHF omnidirectional radio range (VOR) checkpoints | NIL |
| 5 | INS checkpoints | See PRKG-1 chart. |
| 6 | Remarks | NIL |

## SURFACE MOVEMENT GUIDANCE AND CONTROL SYSTEM AND MARKINGS

| 1 | Use of aircraft stand ID signs, TWY guide lines and visual docking/parking guidance system of aircraft stands | NIL |
| --- | --- | --- |
| 2 | RWY and TWY markings and LGT | RWY: NALS, NIL. TWY: NIL |
| 3 | Stop bars | NIL |
| 4 | Other runway protection measures | NIL |
| 5 | Remarks | NIL |

## AERODROME OBSTACLES

For availability and usage of obstacle and terrain data, refer to AD 2.23 Additional Information 3.

| Obstacles Penetrating OLS Inner Horizontal Surface and Conical Surface |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OBST ID/ Designation | OBST type | OBST position | ELEV/HGT | Markings/ Type, colour, lighting (LGT) | Remarks |
| a | b | c | d | e | f |
| BOLU-HGM-0002 | TOWER | 404544.36N 0313703.75E | 865/60 M 2838/197 FT | NIL | Inner Horizontal Surface |
| BOLU-HGM-0003 | TOWER | 404556.25N 0313756.44E | 869/70.1 M 2851/230 FT | NIL | Inner Horizontal Surface |
| BOLU-HGM-0004 | TOWER | 404606.91N 0313931.36E | 897/60 M 2943/197 FT | NIL | Inner Horizontal Surface |
| BOLU-HGM-0017 | TOWER | 404556.23N 0313756.81E | 872/50 M 2861/164 FT | NIL | Inner Horizontal Surface |
| BOLU-IBO-0001 | LIGHT_STANCHION | 404408.85N 0313628.32E | 796.8/64.9 M 2614/213 FT | NIL | Inner Horizontal Surface |
| BOLU-IBO-0002 | LIGHT_STANCHION | 404407.41N 0313622.84E | 798/64.9 M 2618/213 FT | NIL | Inner Horizontal Surface |
| BOLU-IBO-0003 | LIGHT_STANCHION | 404412.85N 0313626.66E | 795.8/64.9 M 2611/213 FT | NIL | Inner Horizontal Surface |
| BOLU-IBO-0004 | LIGHT_STANCHION | 404411.81N 0313621.29E | 797.4/64.9 M 2616/213 FT | NIL | Inner Horizontal Surface |

| Obstacles Penetrating OLS Approach Surfaces and Transitional Surfaces |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OBST ID/ Designation | OBST type | OBST position | ELEV/HGT | Markings/ Type, colour, lighting (LGT) | Remarks |
| a | b | c | d | e | f |
| BOLU-IBO-0055 | BUILDING | 404443.18N 0313822.30E | 754.7/4.3 M 2476/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0056 | BUILDING | 404443.66N 0313822.20E | 754.7/4.3 M 2476/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0057 | BUILDING | 404440.19N 0313823.51E | 753.2/4.3 M 2471/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0058 | BUILDING | 404440.71N 0313823.37E | 753.2/4.3 M 2471/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0059 | BUILDING | 404441.09N 0313823.03E | 754.1/4.3 M 2474/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0060 | BUILDING | 404441.67N 0313822.90E | 754.1/4.3 M 2474/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0061 | BUILDING | 404442.09N 0313822.57E | 754.4/4.3 M 2475/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0062 | BUILDING | 404442.71N 0313822.38E | 754.4/4.3 M 2475/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0063 | BUILDING | 404444.17N 0313822.10E | 755/4.3 M 2477/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0070 | BUILDING | 404436.13N 0313827.95E | 751/4.3 M 2464/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0071 | BUILDING | 404435.84N 0313826.90E | 751.6/4.3 M 2466/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0072 | BUILDING | 404435.40N 0313825.75E | 751.6/4.3 M 2466/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0073 | BUILDING | 404435.10N 0313824.48E | 750.4/4.3 M 2462/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0074 | BUILDING | 404434.60N 0313822.93E | 749.2/4.3 M 2458/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0076 | BUILDING | 404434.26N 0313821.74E | 748.6/4.3 M 2456/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0077 | BUILDING | 404435.79N 0313820.58E | 748.9/4.3 M 2457/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0080 | BUILDING | 404436.93N 0313828.13E | 750.1/4.3 M 2461/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0082 | BUILDING | 404437.18N 0313828.75E | 750.1/4.3 M 2461/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0083 | BUILDING | 404436.56N 0313826.28E | 751.3/4.3 M 2465/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0084 | BUILDING | 404435.79N 0313823.95E | 749.5/4.3 M 2459/14 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0118 | BUILDING | 404430.99N 0313814.18E | 755.6/12.2 M 2479/40 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0119 | BUILDING | 404430.31N 0313811.52E | 755.6/12.2 M 2479/40 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0156 | BUILDING | 404430.53N 0313814.33E | 755.6/12.2 M 2479/40 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0157 | BUILDING | 404429.89N 0313811.70E | 755.6/12.2 M 2479/40 FT | NIL | Approach Surface RWY 06 |
| BOLU-IBO-0013 | BUILDING | 404501.42N 0313905.67E | 748.3/4.3 M 2455/14 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0017 | BUILDING | 404445.41N 0313822.23E | 755/4.3 M 2477/14 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0018 | BUILDING | 404445.03N 0313821.77E | 755/4.3 M 2477/14 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0019 | BUILDING | 404445.64N 0313821.55E | 755/4.3 M 2477/14 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0042 | BUILDING | 404458.19N 0313930.02E | 741.9/4.3 M 2434/14 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0064 | BUILDING | 404444.68N 0313821.98E | 755/4.3 M 2477/14 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0067 | BUILDING | 404436.68N 0313831.51E | 748.9/4.3 M 2457/14 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0068 | BUILDING | 404436.58N 0313830.32E | 749.5/4.3 M 2459/14 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0069 | BUILDING | 404436.41N 0313829.11E | 750.1/4.3 M 2461/14 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0078 | BUILDING | 404437.19N 0313832.14E | 748.6/4.3 M 2456/14 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0086 | BUILDING | 404435.32N 0313830.89E | 751.9/4.3 M 2467/14 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0087 | BUILDING | 404434.79N 0313829.82E | 753.2/4.3 M 2471/14 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0089 | BUILDING | 404434.14N 0313827.72E | 754.1/4.3 M 2474/14 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0090 | BUILDING | 404433.79N 0313826.55E | 754.1/4.3 M 2474/14 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0091 | BUILDING | 404433.40N 0313825.45E | 752.9/4.3 M 2470/14 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0165 | BUILDING | 404440.56N 0313840.55E | 744.9/4.3 M 2444/14 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0177 | BUILDING | 404445.64N 0313856.35E | 745.5/6.1 M 2446/20 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0178 | BUILDING | 404445.52N 0313855.67E | 745.5/6.1 M 2446/20 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0182 | BUILDING | 404445.70N 0313856.72E | 745.2/6.1 M 2445/20 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0191 | BUILDING | 404448.72N 0313904.22E | 743.4/7.6 M 2439/25 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0192 | BUILDING | 404448.86N 0313905.06E | 743.4/7.6 M 2439/25 FT | NIL | Transitional Surface RWY 06 |
| BOLU-IBO-0013 | BUILDING | 404501.42N 0313905.67E | 748.3/4.3 M 2455/14 FT | NIL | Transitional Surface RWY 24 |
| BOLU-IBO-0042 | BUILDING | 404458.19N 0313930.02E | 741.9/4.3 M 2434/14 FT | NIL | Transitional Surface RWY 24 |
| BOLU-IBO-0067 | BUILDING | 404436.68N 0313831.51E | 748.9/4.3 M 2457/14 FT | NIL | Transitional Surface RWY 24 |
| BOLU-IBO-0078 | BUILDING | 404437.19N 0313832.14E | 748.6/4.3 M 2456/14 FT | NIL | Transitional Surface RWY 24 |
| BOLU-IBO-0165 | BUILDING | 404440.56N 0313840.55E | 744.9/4.3 M 2444/14 FT | NIL | Transitional Surface RWY 24 |
| BOLU-IBO-0177 | BUILDING | 404445.64N 0313856.35E | 745.5/6.1 M 2446/20 FT | NIL | Transitional Surface RWY 24 |
| BOLU-IBO-0178 | BUILDING | 404445.52N 0313855.67E | 745.5/6.1 M 2446/20 FT | NIL | Transitional Surface RWY 24 |
| BOLU-IBO-0182 | BUILDING | 404445.70N 0313856.72E | 745.2/6.1 M 2445/20 FT | NIL | Transitional Surface RWY 24 |
| BOLU-IBO-0191 | BUILDING | 404448.72N 0313904.22E | 743.4/7.6 M 2439/25 FT | NIL | Transitional Surface RWY 24 |
| BOLU-IBO-0192 | BUILDING | 404448.86N 0313905.06E | 743.4/7.6 M 2439/25 FT | NIL | Transitional Surface RWY 24 |

| Obstacles Penetrating OLS Take-off Climb Surfaces |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OBST ID/ Designation | OBST type | OBST position | ELEV/HGT | Markings/ Type, colour, lighting (LGT) | Remarks |
| a | b | c | d | e | f |
| BOLU-IBO-0049 | BUILDING | 404440.52N 0313823.86E | 753.2/4.3 M 2471/14 FT | NIL | RWY 24 |
| BOLU-IBO-0050 | BUILDING | 404441.45N 0313823.36E | 754.1/4.3 M 2474/14 FT | NIL | RWY 24 |
| BOLU-IBO-0051 | BUILDING | 404442.53N 0313822.91E | 754.4/4.3 M 2475/14 FT | NIL | RWY 24 |
| BOLU-IBO-0057 | BUILDING | 404440.19N 0313823.51E | 753.2/4.3 M 2471/14 FT | NIL | RWY 24 |
| BOLU-IBO-0058 | BUILDING | 404440.71N 0313823.37E | 753.2/4.3 M 2471/14 FT | NIL | RWY 24 |
| BOLU-IBO-0059 | BUILDING | 404441.09N 0313823.03E | 754.1/4.3 M 2474/14 FT | NIL | RWY 24 |
| BOLU-IBO-0060 | BUILDING | 404441.67N 0313822.90E | 754.1/4.3 M 2474/14 FT | NIL | RWY 24 |
| BOLU-IBO-0061 | BUILDING | 404442.09N 0313822.57E | 754.4/4.3 M 2475/14 FT | NIL | RWY 24 |
| BOLU-IBO-0062 | BUILDING | 404442.71N 0313822.38E | 754.4/4.3 M 2475/14 FT | NIL | RWY 24 |
| BOLU-IBO-0077 | BUILDING | 404435.79N 0313820.58E | 748.9/4.3 M 2457/14 FT | NIL | RWY 24 |
| BOLU-IBO-0119 | BUILDING | 404430.31N 0313811.52E | 755.6/12.2 M 2479/40 FT | NIL | RWY 24 |

| In Area 2a |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OBST ID/ Designation | OBST type | OBST position | ELEV/HGT | Markings/ Type, colour, lighting (LGT) | Remarks |
| a | b | c | d | e | f |
| BOLU-IBO-0015 | BUILDING | 404454.39N 0313900.93E | 746.1/4.3 M 2448/14 FT | NIL | In runway strip |
| BOLU-IBO-0047 | BUILDING | 404445.92N 0313843.87E | 746.5/4.3 M 2449/14 FT | NIL | In runway strip, on apron |
| BOLU-IBO-0167 | BUILDING | 404445.92N 0313846.95E | 748.9/6.1 M 2457/20 FT | NIL | In runway strip |

| In Area 2b / Obstacles Penetrating Take-off Flight Path Area |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OBST ID/ Designation | OBST type | OBST position | ELEV/HGT | Markings/ Type, colour, lighting (LGT) | Remarks |
| a | b | c | d | e | f |
| BOLU-IBO-0001 | LIGHT_STANCHION | 404408.85N 0313628.32E | 796.8/64.9 M 2614/213 FT | NIL | RWY 24 |
| BOLU-IBO-0002 | LIGHT_STANCHION | 404407.41N 0313622.84E | 798/64.9 M 2618/213 FT | NIL | RWY 24 |
| BOLU-IBO-0003 | LIGHT_STANCHION | 404412.85N 0313626.66E | 795.8/64.9 M 2611/213 FT | NIL | RWY 24 |
| BOLU-IBO-0004 | LIGHT_STANCHION | 404411.81N 0313621.29E | 797.4/64.9 M 2616/213 FT | NIL | RWY 24 |
| BOLU-IBO-0020 | BUILDING | 404435.40N 0313816.80E | 749.2/4.3 M 2458/14 FT | NIL | RWY 24 |
| BOLU-IBO-0049 | BUILDING | 404440.52N 0313823.86E | 753.2/4.3 M 2471/14 FT | NIL | RWY 24 |
| BOLU-IBO-0050 | BUILDING | 404441.45N 0313823.36E | 754.1/4.3 M 2474/14 FT | NIL | RWY 24 |
| BOLU-IBO-0051 | BUILDING | 404442.53N 0313822.91E | 754.4/4.3 M 2475/14 FT | NIL | RWY 24 |
| BOLU-IBO-0057 | BUILDING | 404440.19N 0313823.51E | 753.2/4.3 M 2471/14 FT | NIL | RWY 24 |
| BOLU-IBO-0058 | BUILDING | 404440.71N 0313823.37E | 753.2/4.3 M 2471/14 FT | NIL | RWY 24 |
| BOLU-IBO-0059 | BUILDING | 404441.09N 0313823.03E | 754.1/4.3 M 2474/14 FT | NIL | RWY 24 |
| BOLU-IBO-0060 | BUILDING | 404441.67N 0313822.90E | 754.1/4.3 M 2474/14 FT | NIL | RWY 24 |
| BOLU-IBO-0061 | BUILDING | 404442.09N 0313822.57E | 754.4/4.3 M 2475/14 FT | NIL | RWY 24 |
| BOLU-IBO-0062 | BUILDING | 404442.71N 0313822.38E | 754.4/4.3 M 2475/14 FT | NIL | RWY 24 |
| BOLU-IBO-0077 | BUILDING | 404435.79N 0313820.58E | 748.9/4.3 M 2457/14 FT | NIL | RWY 24 |
| BOLU-IBO-0116 | BUILDING | 404434.94N 0313805.52E | 751/4.3 M 2464/14 FT | NIL | RWY 24 |
| BOLU-IBO-0117 | BUILDING | 404434.90N 0313804.37E | 751/4.3 M 2464/14 FT | NIL | RWY 24 |
| BOLU-IBO-0119 | BUILDING | 404430.31N 0313811.52E | 755.6/12.2 M 2479/40 FT | NIL | RWY 24 |
| BOLU-IBO-0151 | BUILDING | 404430.53N 0313806.90E | 754.7/12.2 M 2476/40 FT | NIL | RWY 24 |
| BOLU-IBO-0152 | BUILDING | 404430.16N 0313807.12E | 754.7/12.2 M 2476/40 FT | NIL | RWY 24 |

| In Area 3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OBST ID/ Designation | OBST type | OBST position | ELEV/HGT | Markings/ Type, colour, lighting (LGT) | Remarks |
| a | b | c | d | e | f |
| BOLU-IBO-0011 | BUILDING | 404505.44N 0313952.92E | 736.7/4.3 M 2417/14 FT | NIL |  |
| BOLU-IBO-0014 | BUILDING | 404509.62N 0313907.67E | 754.7/4.3 M 2476/14 FT | NIL |  |
| BOLU-IBO-0015 | BUILDING | 404454.39N 0313900.93E | 746.1/4.3 M 2448/14 FT | NIL |  |
| BOLU-IBO-0016 | BUILDING | 404445.61N 0313830.23E | 751.3/4.3 M 2465/14 FT | NIL |  |
| BOLU-IBO-0026 | BUILDING | 404516.04N 0313932.88E | 748.9/4.3 M 2457/14 FT | NIL |  |
| BOLU-IBO-0027 | BUILDING | 404516.79N 0313922.43E | 756.5/4.3 M 2482/14 FT | NIL |  |
| BOLU-IBO-0029 | BUILDING | 404517.77N 0313920.44E | 757.7/4.3 M 2486/14 FT | NIL |  |
| BOLU-IBO-0030 | BUILDING | 404517.85N 0313920.15E | 757.7/4.3 M 2486/14 FT | NIL |  |
| BOLU-IBO-0047 | BUILDING | 404445.92N 0313843.87E | 746.5/4.3 M 2449/14 FT | NIL |  |
| BOLU-IBO-0052 | BUILDING | 404443.48N 0313822.78E | 754.7/4.3 M 2476/14 FT | NIL |  |
| BOLU-IBO-0053 | BUILDING | 404444.48N 0313822.47E | 755/4.3 M 2477/14 FT | NIL |  |
| BOLU-IBO-0056 | BUILDING | 404443.66N 0313822.20E | 754.7/4.3 M 2476/14 FT | NIL |  |
| BOLU-IBO-0063 | BUILDING | 404444.17N 0313822.10E | 755/4.3 M 2477/14 FT | NIL |  |
| BOLU-IBO-0064 | BUILDING | 404444.68N 0313821.98E | 755/4.3 M 2477/14 FT | NIL |  |
| BOLU-IBO-0067 | BUILDING | 404436.68N 0313831.51E | 748.9/4.3 M 2457/14 FT | NIL |  |
| BOLU-IBO-0068 | BUILDING | 404436.58N 0313830.32E | 749.5/4.3 M 2459/14 FT | NIL |  |
| BOLU-IBO-0069 | BUILDING | 404436.41N 0313829.11E | 750.1/4.3 M 2461/14 FT | NIL |  |
| BOLU-IBO-0070 | BUILDING | 404436.13N 0313827.95E | 751/4.3 M 2464/14 FT | NIL |  |
| BOLU-IBO-0078 | BUILDING | 404437.19N 0313832.14E | 748.6/4.3 M 2456/14 FT | NIL |  |
| BOLU-IBO-0080 | BUILDING | 404436.93N 0313828.13E | 750.1/4.3 M 2461/14 FT | NIL |  |
| BOLU-IBO-0081 | BUILDING | 404437.73N 0313831.31E | 748.3/4.3 M 2455/14 FT | NIL |  |
| BOLU-IBO-0082 | BUILDING | 404437.18N 0313828.75E | 750.1/4.3 M 2461/14 FT | NIL |  |
| BOLU-IBO-0108 | BUILDING | 404444.83N 0313846.63E | 745.9/4.3 M 2447/14 FT | NIL |  |
| BOLU-IBO-0109 | BUILDING | 404447.91N 0313855.05E | 744/4.3 M 2441/14 FT | NIL |  |
| BOLU-IBO-0167 | BUILDING | 404445.92N 0313846.95E | 748.9/6.1 M 2457/20 FT | NIL |  |
| BOLU-IBO-0168 | BUILDING | 404446.04N 0313847.64E | 748.9/6.1 M 2457/20 FT | NIL |  |
| BOLU-IBO-0173 | BUILDING | 404446.27N 0313848.89E | 748.6/6.1 M 2456/20 FT | NIL |  |
| BOLU-IBO-0174 | BUILDING | 404446.13N 0313848.21E | 748.6/6.1 M 2456/20 FT | NIL |  |

## METEOROLOGICAL INFORMATION PROVIDED

| 1 | Associated MET office | NIL |
| --- | --- | --- |
| 2 | Hours of service MET office outside hours | NIL |
| 3 | Office responsible for terminal aerodrome forecast (TAF) preparation Periods of validity | NIL |
| 4 | Trend forecast Interval of issuance | NIL |
| 5 | Briefing/consultation provided | NIL |
| 6 | Flight documentation Language(s) used | NIL |
| 7 | Charts and other information available for briefing or consultation | NIL |
| 8 | Supplementary equipment available for providing information | NIL |
| 9 | ATS units provided with information | NIL |
| 10 | Additional information (limitation of service, etc.) | Refer to AD 2.22 Flight procedures 8. for availability and use of meteorological information. |

## RUNWAY PHYSICAL CHARACTERISTICS

| Designations RWY NR | TRUE BRG | Dimensions of RWY (M) | Strength of the pavement classification rating (PCR) and surface of RWY and SWY | THR coordinates RWY end coordinates THR geoid undulation | THR elevation and highest elevation of TDZ of precision APP RWY |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06 | 64.62° | 1 895 × 45 | Unknown Concrete | 404441.93N 0313829.94E GUND -38 M | THR 744 M/ 2441 FT |
| 24 | 244.62° | 1 895 × 45 | Unknown Concrete | 404508.21N 0313943.02E GUND -38 M | THR 737 M/ 2416 FT |
| Designations RWY NR | Slope of RWY-SWY | SWY dimensions (M) | Clearway (CWY) dimensions (M) | Strip dimensions (M) | Dimensions of runway end safety areas |
| 1 | 7 | 8 | 9 | 10 | 11 |
| 06 | -0.39%* | NIL | 130 × 280*** | 1 895 × 120*** | 130 x 120*** |
| 24 | +0.39%** | NIL | 90 × 230*** | 1 895 × 120*** | 90 x 120*** |

| Designations RWY NR | Location and description of engineering material arresting system (EMAS) | OFZ | Remarks |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | 12 | 13 | 14 |  |  |
| 06 | NIL | Not applicable. | * -0.62% (0-976 m), +0.07% (976-1281 m), -0.17% (1281-1342 m), +0.07% (1342-1525 m), -0.45 % (1525-1895 m) ***Small trees may be present in the area. |  |  |
| 24 | NIL | Not applicable. | ** +0.45% (0-370 m), -0.07% (370-553 m), +0.17% (553-614 m), -0.07% (614-919 m), +0.62% (919-1895 m) ***Small trees may be present in the area. |  |  |

## DECLARED DISTANCES

| RWY designator | TORA (M) | TODA (M) | ASDA (M) | LDA (M) | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 06 | 1 895 | 2 025 | 1 895 | 1 895 | NIL |
| 24 | 1 895 | 1 985 | 1 895 | 1 895 | NIL |

## APPROACH AND RUNWAY LIGHTING

| RWY designator | APCH LGT type LEN INTST | THR LGT colour WBAR | VASIS (MEHT) PAPI | TDZ, LGT LEN | RWY Centre Line LGT Length, spacing, colour, INTST | RWY edge LGT LEN, spacing colour INTST | RWY End LGT colour WBAR | SWY LGT LEN (M) colour | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| 06 | NALS | NIL – | NIL | NIL | NIL | NIL | NIL – | NIL | NIL |
| 24 | NALS | NIL – | NIL | NIL | NIL | NIL | NIL – | NIL | NIL |

## OTHER LIGHTING, SECONDARY POWER SUPPLY

| 1 | ABN/IBN location, characteristics and hours of operation | ABN: NIL/IBN: NIL NIL |
| --- | --- | --- |
| 2 | LDI location and LGT Anemometer location and LGT | LDI: NIL Anemometer: NIL |
| 3 | TWY edge lights, centre line lights and stop bars (if any) | Edge: NIL Centre line: NIL Stop bars: NIL |
| 4 | Secondary power supply/switch-over time | NIL Switch-over time: NIL |
| 5 | Remarks | NIL |

## HELICOPTER LANDING AREA

NIL

## ATS AIRSPACE

| 1 | Designation and lateral limits | BOLU ATZ 404453N 0314628E, 404923N 0314333E, 404457N 0313145E, 404027N 0313440E |
| --- | --- | --- |
| 2 | Vertical limits | SFC to 5 500 FT AMSL |
| 3 | Airspace classification | D |
| 4 | ATS unit call sign Language(s) | Bolu Tower Turkish, English |
| 5 | Transition altitude | 10 000 FT AMSL |
| 6 | Hours of applicability (or activation) | H24 |
| 7 | Remarks | ATS airspace available H24, ATS unit O/R. |

## ATS COMMUNICATION FACILITIES

| Service designation | Call sign | Frequency | Hours of operation | Remarks |
| --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 |
| TWR | Bolu Tower | 122.100 MHz 121.500 MHz* | O/R** | *Emergency FREQ. **Applicable when mobile service is established. |
| PAR | Bolu Precision | 123.300 MHz*** 121.500 MHz* | O/R** | *Emergency FREQ. **Applicable when mobile service is established. ***EUR common PAR channel. |
| NIL | NIL | 123.500 MHz**** | H24 | ****EUR regional light aviation channel. |

## RADIO NAVIGATION AND LANDING AIDS

| Type of aid, MAG VAR, Type of supported OPS for ILS/MLS/GLS, Basic GNSS/SBAS, Classification for ILS, Facility classification and APP facility designator(s) for GBAS, Station declination for VOR/ILS/MLS | ID | FREQ, CH NR, Service provider, RPI | Hours of OPS | Position of transmitting antenna coordinates | ELEV of DME transmitting antenna | Service volume radius from the GBAS reference point | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| GPS |  | 1575.42 MHz | H24 |  |  |  |  |

## LOCAL AERODROME REGULATIONS

### Applicability of this Aeronautical Information Publication

This aeronautical information publication has not been issued by a valid air navigation service provider and carries no official status. It is also not affiliated with the aerodrome administration. The use of this publication is solely the responsibility of the operator or the pilot-in-command. Appropriate assessments should be conducted prior to use.

### Application of rules and regulations

Except as otherwise specified in other paragraphs, the implementation of rules, procedures and design criteria are not governed by the Türkiye AIP.

Except where otherwise specified; ICAO rules, Standards and Recommended Practices (SARPs) and design criteria, together with differences and additions specified in NATO standards for military procedures, and EUROCONTROL regulations for flow management, shall apply.

Aircraft operations are based on SHGM (DGCA of Türkiye) and NATO regulations.

### Authorization of aerodrome usage

Aerodrome located in General Eşlef Bitlis Kışlası and prior authorization is required for use. Please contact with General Eşlef Bitlis Kışlası Administration for prior authorization.

### Ground operations

#### Details and usage of taxiways

The sections of TWY A that are farther from the runway are not suitable for use due to present trees.

The sections of TWY A adjacent to the runway (TWY A1 and TWY A2 sides) may be used for parking purposes when APN 1 is fully occupied, when the aircraft cannot be accommodated within APN 1, or on request.

Bearing strength of pavement of TWY is unknown. The evaluation of TWY bearing strength is the responsibility of the operator.

#### Details and usage of aprons

There is a building in the middle of the APN 1.

APN 1 is limited in size and may not be able to accommodate certain aircraft. In such cases, if acceptable for the aircraft, the aircraft may extend beyond the apron boundaries onto unpaved ground, taxiways may be used for parking purposes, or, if no other traffic is expected to use the runway, the aircraft may be parked with part of the aircraft extending onto the runway.

Bearing strength of pavement of APN is unknown. The evaluation of APN bearing strength is the responsibility of the operator.

#### Details and usage of runways

On runway, two white ‘H’ markings of 12 m × 8 m are located on the runway centreline at distances of 43 m and 125 m from the RWY 06 threshold. These markings have no specific operational purpose, such as a designated helicopter landing area.

Bearing strength of pavement of RWY is unknown. The evaluation of RWY bearing strength is the responsibility of the operator.

### Flight planning considerations

Flight planning requirements described in AIP Türkiye ENR 1.10 Flight Planning are applicable for Bolu AD. In this context, Bolu AD is not covered by the AIP Türkiye and does not have a valid ICAO Location Indicator; therefore, the relevant instructions shall be applied.

Bolu AD does not exist in the EUROCONTROL IFPS database so IFP flight plan filling rules for non-designated aerodromes shall be apply for IFR flight plans.

For flight planning guidelines, refer to AD 2.23 Additional Information 5.

## NOISE ABATEMENT PROCEDURES

### Application of noise abatement departure procedure (NADP)

#### ACFTs departing from RWY 06

ACFTs in compliance with the noise category specified in ICAO Annex 16, Volume I, Chapters 2, 3, 4, 5, 12 and 14, departing from RWY 06 shall apply NADP 2 until passing 3000 FT AAL (for abating noise at far areas), except in cases where NADP may not be applied under ICAO Doc 8168 PANS-OPS Volume III.

#### ACFTs departing from RWY 24

ACFTs in compliance with the noise category specified in ICAO Annex 16, Volume I, Chapters 2, 3, 4, 5, 12 and 14, departing from RWY 24 shall apply NADP 1 until passing 3000 FT AAL (for abating noise at near areas, city), except in cases where NADP may not be applied under ICAO Doc 8168 PANS-OPS Volume III.

### Noise preferential runways

RWY 06 is the noise preferential runway for take-off.

Noise preferential runway shall not be a determining factor in runway nomination under the provisions in ICAO Doc 4444 and Doc 8168 Volume III. Briefly:

- pilot-in-command prompted by safety concerns can refuse a runway offered for noise preferential reasons;

- if the runway surface conditions are adversely affected (e.g. by snow, slush, ice, water, mud, rubber, oil or other substances);

- for take-off when the visibility is less than 1 900 m;

- when wind shear has been reported or forecast or when thunderstorms are expected to affect the approach or departure;

- when the crosswind component, including gusts, exceeds 15 kt (28 km/h), or the tailwind component, including gusts, exceeds 5 kt (9 km/h).

For selection of runway in use, refer to AD 2.22 Flight procedures 7. f.

## FLIGHT PROCEDURES

### General

All flights shall obey to rules described in ICAO Annex 2 – Rules of the air.

### Designation of AD and RWY

- Aerodrome Reference Code of aerodrome is 3C.

- Runway 06/24 is designated as Instrument Runway – Non-precision Approach Runway.

### Designation as PIC/commander – aerodrome knowledge categorization (EASA AIR-OPS)

SHGM applies EASA  Air-Ops – Designation as PIC/commander – Aerodrome knowledge categorization (AMC1 ORO.FC.105(b)(2);(c)). Under this regulation, Bolu AD is classified as Category B. Any commercial air transport operator that plans to conduct landing and take-off operations to such airport should meet this requirement.

### Instrument flight rules

IFR flights shall obey the IFR rules described in ICAO Annex 2.

IFR SID, STAR and IAP procedures are not established yet. IFR arrivals may be planned for PAR approaches, with the required obstacle clearance maintained under the responsibility of the pilot-in-command and the operator.

### Visual flight rules

VFR flights shall obey the VFR rules described in ICAO Annex 2.

VFR flights shall apply ATM related procedures in accordance with the airspace classification requirements of Bolu ATZ and except for the differences specified in this AIP, in accordance with the relevant ICAO and NATO regulations. (Such as ICAO Annex 10, Annex 11, Doc 4444 and Doc 9432; NATO AATMP-06 and AATCP-2.)

VFR flights cannot be initiated in night time except for the appropriate authorization taken from ATS unit which processes the flight plan.

### Night time

For determination of night time, same time interval described in AIP Türkiye ENR 1.2 Visual flight rules shall be used.

Note: Night will be considered the period between 30 minutes after sunset and 30 minutes before sunrise as described by AIP Türkiye ENR 1.2.

Tables given in AD 2.23 Additional information 7. can be used for determining sunrise and sunset times.

### Air traffic management

#### ATM when TWR is not available

When TWR is not available, all traffic within Bolu ATZ shall monitor 123.500 MHz EUR regional light aviation channel and shall use this frequency for position reporting and communication to ensure traffic separation and flight safety. The following should be reported, with phraseologies described in ICAO Doc 4444, 9432 and NATO AATCP-2, if available:

- position reports at compulsory reporting points;

- entering to or leaving from traffic circuit or NATO traffic patterns;

- position reports on traffic circuit or NATO traffic patterns;

- on final approach;

- on landing to or departing from runway, vacating from runway.

Air traffic control service is not provided; therefore, IFR traffic shall be responsible for maintaining separation from other traffic, irrespective of the airspace classification.

#### ATM when TWR is available

When mobile TWR unit is established, aerodrome control procedures shall be applied in accordance with ICAO Doc 4444.

#### PAR service

When mobile PAR unit is established, PAR service can be used for landings.

PAR phraseology used for military operations should include differences and additions specified in NATO AATCP-2 in addition to ICAO phraseology. For civil operations, only ICAO phraseology should be used.

PAR IAP’s not established yet.

#### Use of Turkish Language

When the Turkish Language is used, phraseology should be in accordance with book of the DHMİ Havacılık Frezyolojisi. If this resource does not satisfy the requirements specified in other provisions, or if the use of plain language disrupts the systematic consistency, the use of English phraseology in accordance with the relevant sources is recommended.

#### Radio communication failure

As a pilot discretion, decide to land to Bolu AD or initiate diverting to another suitable AD. Use procedures and phraseologies described in ICAO Annex 10, Doc 4444, Doc 9432; NATO AATCP-2.

#### Selection of runway-in-use

Selection of the runway-in-use shall be made as follows:

- if ATC service is available, the runway shall be selected by ATC but pilot-in-command however, the pilot-in-command may request the use of a different runway due to operational difficulties, performance limitations, or safety concerns;

- if ATC service is not available, the pilot-in-command shall select the runway, if necessary in consultation with other traffic, and shall inform the other traffic accordingly.

Runway nomination presented for noise abatement purposes. If able, the noise preferential runway shall be selected. Refer to AD 2.21 Noise abatement procedures 2. For cases where the noise abatement procedures cannot be applied or are not covered by the runway nomination, the following item applies.

Selection of runway-in-use shall be carried out in accordance with the procedures specified in ICAO Doc 4444, briefly:

- normally, an aircraft will land and take off into wind unless safety, the runway configuration, meteorological conditions and available instrument approach procedures or air traffic conditions determine that a different direction is preferable. In selecting the runway-in-use, however, the unit providing aerodrome control service shall take into consideration, besides surface wind speed and direction, other relevant factors such as the aerodrome traffic circuits, the length of runways, and the approach and landing aids available.

### Availability and use of meteorological information

Bolu AD does not have a MET office and appropriate meteorological measuring equipment. Therefore, general meteorological information and altimeter setting information cannot be provided.

If the tower is operational, the necessary mobile measuring equipment is available, and the tower is capable of providing meteorological information, the meteorological information provided by the tower may be used.

If meteorological information cannot be provided by tower or appropriate ways, weather information for the city of Bolu published on the Meteoroloji Genel Müdürlüğü (MGM) website may be used for supplementary purposes only and not for operational use.

Website: mgm.gov.tr

### Altimeter Setting Procedures

#### Operations when altimeter setting information is available

If the tower can provide altimeter setting information, the QNH and QFE values reported by the tower can be used for altimeter setting purposes.

#### Operations when altimeter setting information is not available

For departure, en-route or arrival phases of flight, or sections of instrument approaches not related to RASS, the responsibility for establishing a safe altimeter setting for ensuring adequate terrain or obstacle clearance, or for other altimeter-setting-related purposes, to the extent covered in ICAO PANS-OPS Volume III and, where necessary, with additional considerations, using actual or forecast meteorological information, including QNH reports or forecast lowest mean sea level pressure, obtained from MET offices of surrounding aerodromes or other available local sources, rests with the operator or the pilot-in-command.

For PANS-OPS/MIPS based instrument approach operations, Remote Altimeter Setting Source (RASS) may be used. Where an instrument approach procedure is based on RASS, the relevant information will be stated on the relevant chart.

Surrounding aerodromes that may be used as remote altimeter setting source for instrument approach procedures in accordance with ICAO PANS-OPS are listed below.

| Surrounding Aerodromes for Use as RASS |  |  |  |  |
| --- | --- | --- | --- | --- |
| ICAO location indicator | Name of aerodrome | Elevation of AD | Distance from Bolu AD ARP | ELEV difference from Bolu AD ELEV |
| LTAS | ZONGULDAK / Çaycuma | 44 ft | 49.7 NM | -2397 ft |
| LTAE | ANKARA / Mürted | 2767 ft | 58.0 NM | +326 ft |
| LTBQ | KOCAELİ / Cengiz Topel | 188 ft | 71.3 NM | -2253 ft |
| LTAD | ANKARA / Etimesgut | 2653 ft | 67.4 NM | +212 ft |
| LTAB | ANKARA / Güvercinlik | 2691 ft | 69.7 NM | +250 ft |
| LTAC | ANKARA / Esenboğa | 3158 ft | 71.8 NM | +717 ft |

#### Reference pressure

For convenience, the atmospheric pressure corresponding to the aerodrome elevation and the pressure difference have been calculated. These values may be useful for various calculations:

- reference atmospheric pressure for AD ELEV is calculated as 927 hPa under ISA conditions;

- difference from MSL atmospheric pressure is 86.3 hPa under ISA conditions.

#### Determination of transition altitude (TA) and transition level (TRL)

Transition altitude is established as 12000 FT AMSL and published in AD 2.17 ATS Airspace 5. for compatibility with surrounding aerodromes. Transition level shall be determined in accordance with AIP Türkiye ENR 1.7 Altimeter setting procedures for compatibility with surrounding aerodromes. The following transition level determination table is applicable to Bolu AD in accordance with the relevant provisions:

| QNH (HPA) TA (AMSL) | Corresponding Transition Level (TL) by QNH Ranges |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 940-946 | 947-962 | 963-979 | 980-996 | 997-1012 | 1013-1029 | 1030-1046 | 1047-1050 |
| 12000 FT | FL 155 | FL 150 | FL 145 | FL 140 | FL 135 | FL 130 | FL 125 | FL 120 |

### Aerodrome operating minima (AOM)

#### Application of take-off and landing minima

Both SHGM SHT-OPS and NATO AATCP-1 minimums shall apply at Bolu AD. Ibosoft may provide predefined take-off and landing minimums for guidance, without any aircraft, aircrew or operator-related adjustments. These minimums may be used directly if no additional adjustments are required, or with the necessary adjustments applied in accordance with the applicable AOM policy.

In the minima boxes, SHGM SHT-OPS based minimums are labelled as “SHT-OPS”. NATO AATCP-1 based minimums are labelled as “MIPS”

Take-off minimums are published on the ADC chart. Approach minima are published on the associated charts, where applicable.

## ADDITIONAL INFORMATION

### Source of aeronautical information

AIP and AIS information for Bolu AD is provided by Ibosoft via Ibosoft AIS.

Website: ais.ibosoft.net.tr

### Abbreviations used in this document

Abbreviations are selected from the relevant sources as appropriate to the subject matter. For ICAO design-based content, ICAO abbreviations are used, primarily in accordance with ICAO Doc 8400 PANS-ABC. For non-ICAO-related content, abbreviations associated with the relevant sources are used. For source-based definitions of abbreviations, refer to the Ibosoft Learning Website – Abbreviations Database.

Website: egitim.ibosoft.net.tr/abbreviations-database

### Availability and usage of obstacle and terrain data

#### Obstacle data sources and quality

Trees have not been surveyed. Exercise caution regarding trees.

Obstacles with a Designation value prefixed by “BOLU-HGM-” were obtained from raster TFC/SAC charts produced by the Harita Genel Müdürlüğü (HGM). Top elevation and height values directly taken from related maps. Their positions are relatively consistent. However, their type and marking/lighting status are unknown.

Obstacles with a Designation value prefixed by “BOLU-IBO-” were derived from satellite and street imagery. Their bottom elevations are based on elevation data from HGM or Google Earth. Vertical extent values were estimated based on factors such as building floor counts, obstacle shadow lengths. These obstacles are not consistent.

#### Terrain data sources and quality

Runway profile is derived from elevation data of HGM. Runway related elevations are relatively consistent, but resolution is unknown.

Terrain assessment outside of the AD area is based on SRTM 1 arc-second DEM data. Due to this data being provided in approximately 30 m tiles, terracing artifacts may occur in terrain on some charts.

#### Provided obstacle data

On AOC Type A and AOC Type B charts, obstacles penetrating the relevant obstacle limitation surfaces are depicted. In the AD 2.10 Aerodrome obstacles section, obstacles penetrating the surfaces or areas specified in the respective table headings are listed.

In addition, a digital obstacle data set compatible with the AIXM 5.2 format is provided through Ibosoft AIS. (See AD 2.23 Additional information 1.) Along with the obstacles shown on the AOC charts and in AD 2.10, this data set may also contain surveyed obstacles that do not penetrate the relevant assessment surfaces or areas specified therein.

#### Deviations from ICAO standards in AOC charts

On the AOC Type A – RWY 24 (AOC-2) chart, sections that could not be shown in sufficient detail within the chart scale are additionally presented as separate insets.

Furthermore, since the RWY 24 take-off flight path area passes over the city and sufficient obstacle data for the urban area was not available, certain estimated areas that may contain obstacles penetrating the surface have been indicated on AOC Type A – RWY 24 (AOC-2) chart. These estimations were made by considering the maximum permitted building floor limits for different urban zones together with the terrain elevations of those zones. The symbology used is described on the relevant chart.

Due to the wide distribution of terrain and obstacles penetrating the OLS, the AOC Type B charts would have required excessively large paper sizes within the scale ranges specified in ICAO Annex 4. Therefore, the charts were published in several sections, deviating from certain provisions of ICAO Annex 4.

### Charting and Geographical Basis

#### Charting Criteria and Data Sources

Charting criteria and chart symbols are based on ICAO provisions and SARPs (ICAO Annex 4, Annex 15, Doc 8697, and Doc 10066 PANS-AIM). Additional differences are described on relevant charts or in relevant AIP sections.

Aeronautical data presented on charts, excluding data originating from AIP Ibosoft, are based on AIP Türkiye. For details regarding such data, refer to AIP Türkiye.

#### GEN 2.1.3. Horizontal reference system

- 3.1 Name/designation of system: All published geographical coordinates indicating latitude and longitude are expressed in terms of the World Geodetic System — 1984 (WGS-84) geodetic reference datum.

- 3.2 Parameters of the Projection: Projection is expressed in term of Universal Transverse Mercator (UTM).

- 3.3 Ellipsoid: Ellipsoid is expressed in terms of the World Geodetic System — 1984 (WGS-84) ellipsoid.

- 3.4 Datum: The World Geodetic System — 1984 (WGS-84) is used.

- 3.5 Area of application: NIL

- 3.6 Use of an asterisk to identify published geographical coordinates: NIL

#### GEN 2.1.4. Vertical reference system

- 4.1 Name/designation of system: The vertical reference system corresponds to mean sea level (MSL).

- 4.2 Geoid model: The geoid model used is the Earth Gravitational Model 1996 — (EGM-96)

#### Other

PDF versions of charts containing geographic data support geolocation.

### Flight plan filling guidelines

This guideline describes FPL filling for Bolu AD accordance with ICAO Doc 4444, AIP Türkiye ENR 1.10, EUROCONTROL IFPS User Manual.

#### Addressing of the flight plans

Refer to AIP Türkiye ENR 1.10.

#### Filling the departure aerodrome for departing flights

- Due to Bolu AD is not having an ICAO location indicator, insert “ZZZZ” to FPL Item 13 – Departure Aerodrome

- Fill FPL Item 18 – Other Information with inserting “DEP/” as appropriate, for example:
“DEP/BOLU 4045N03139E”

#### Filling the arrival aerodrome for arrival flights

- Due to Bolu AD is not having an ICAO location indicator, insert “ZZZZ” to FPL Item 16 – Destination Aerodrome

- Fill FPL Item 18 – Other Information with inserting “DEST/” as appropriate, for example:
“DEST/BOLU 4045N03139E”

#### Filling the route

- For departing flights, insert “DCT” to beginning of the FPL Item 15 – Route as:
“DCT [First significant point] …”

- For arriving flights, insert “DCT” to end of the FPL Item 15 – Route as:
“… [Last significant point] DCT”

### Utilization of Bolu AD and AIP/AIS data in flight simulation softwares

#### Scenery availability

- Bolu AD add-on scenery for Prepar3D v4 and later is available, developed by Ibosoft. It can be downloaded from this website: www.ibosoft.net.tr/scenery/kucuk-havalimanlari-scenery

- Bolu AD is available by default in Microsoft Flight Simulator 2020 and 2024.

### Sunrise – sunset tables

#### Year 2026 (UTC)

| JANUARY - 2026 |  |  |  |  | FEBRUARY - 2026 |  |  |  |  | MARCH - 2026 |  |  |  |  | APRIL - 2026 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DAY | TWIL FROM | SR | SS | TWIL TO | DAY | TWIL FROM | SR | SS | TWIL TO | DAY | TWIL FROM | SR | SS | TWIL TO | DAY | TWIL FROM | SR | SS | TWIL TO |
| 1 | 0441 | 0513 | 1440 | 1512 | 1 | 0429 | 0500 | 1514 | 1544 | 1 | 0355 | 0424 | 1547 | 1616 | 1 | 0304 | 0334 | 1621 | 1650 |
| 2 | 0441 | 0513 | 1440 | 1513 | 2 | 0428 | 0459 | 1515 | 1545 | 2 | 0353 | 0422 | 1548 | 1617 | 2 | 0303 | 0332 | 1622 | 1651 |
| 3 | 0441 | 0514 | 1441 | 1514 | 3 | 0427 | 0458 | 1516 | 1547 | 3 | 0352 | 0421 | 1549 | 1618 | 3 | 0301 | 0330 | 1623 | 1652 |
| 4 | 0441 | 0514 | 1442 | 1515 | 4 | 0426 | 0457 | 1517 | 1548 | 4 | 0350 | 0419 | 1550 | 1620 | 4 | 0259 | 0329 | 1624 | 1653 |
| 5 | 0441 | 0514 | 1443 | 1516 | 5 | 0425 | 0456 | 1519 | 1549 | 5 | 0349 | 0418 | 1552 | 1621 | 5 | 0257 | 0327 | 1625 | 1655 |
| 6 | 0441 | 0514 | 1444 | 1516 | 6 | 0424 | 0455 | 1520 | 1550 | 6 | 0347 | 0416 | 1553 | 1622 | 6 | 0256 | 0325 | 1626 | 1656 |
| 7 | 0441 | 0513 | 1445 | 1517 | 7 | 0423 | 0454 | 1521 | 1551 | 7 | 0346 | 0415 | 1554 | 1623 | 7 | 0254 | 0324 | 1627 | 1657 |
| 8 | 0441 | 0513 | 1446 | 1518 | 8 | 0422 | 0452 | 1522 | 1552 | 8 | 0344 | 0413 | 1555 | 1624 | 8 | 0252 | 0322 | 1628 | 1658 |
| 9 | 0441 | 0513 | 1447 | 1519 | 9 | 0421 | 0451 | 1523 | 1554 | 9 | 0342 | 0411 | 1556 | 1625 | 9 | 0251 | 0321 | 1629 | 1659 |
| 10 | 0441 | 0513 | 1448 | 1520 | 10 | 0420 | 0450 | 1525 | 1555 | 10 | 0341 | 0410 | 1557 | 1626 | 10 | 0249 | 0319 | 1630 | 1700 |
| 11 | 0441 | 0513 | 1449 | 1521 | 11 | 0419 | 0449 | 1526 | 1556 | 11 | 0339 | 0408 | 1558 | 1627 | 11 | 0247 | 0317 | 1631 | 1701 |
| 12 | 0440 | 0512 | 1450 | 1522 | 12 | 0418 | 0448 | 1527 | 1557 | 12 | 0338 | 0407 | 1559 | 1628 | 12 | 0246 | 0316 | 1632 | 1702 |
| 13 | 0440 | 0512 | 1451 | 1523 | 13 | 0417 | 0446 | 1528 | 1558 | 13 | 0336 | 0405 | 1600 | 1629 | 13 | 0244 | 0314 | 1633 | 1703 |
| 14 | 0440 | 0512 | 1452 | 1524 | 14 | 0415 | 0445 | 1530 | 1559 | 14 | 0334 | 0403 | 1601 | 1630 | 14 | 0243 | 0313 | 1634 | 1705 |
| 15 | 0440 | 0511 | 1453 | 1525 | 15 | 0414 | 0444 | 1531 | 1601 | 15 | 0333 | 0402 | 1603 | 1632 | 15 | 0241 | 0311 | 1635 | 1706 |
| 16 | 0439 | 0511 | 1455 | 1526 | 16 | 0413 | 0443 | 1532 | 1602 | 16 | 0331 | 0400 | 1604 | 1633 | 16 | 0239 | 0310 | 1636 | 1707 |
| 17 | 0439 | 0511 | 1456 | 1527 | 17 | 0412 | 0441 | 1533 | 1603 | 17 | 0329 | 0358 | 1605 | 1634 | 17 | 0238 | 0308 | 1638 | 1708 |
| 18 | 0438 | 0510 | 1457 | 1528 | 18 | 0410 | 0440 | 1534 | 1604 | 18 | 0328 | 0357 | 1606 | 1635 | 18 | 0236 | 0307 | 1639 | 1709 |
| 19 | 0438 | 0510 | 1458 | 1530 | 19 | 0409 | 0439 | 1535 | 1605 | 19 | 0326 | 0355 | 1607 | 1636 | 19 | 0235 | 0305 | 1640 | 1710 |
| 20 | 0437 | 0509 | 1459 | 1531 | 20 | 0408 | 0437 | 1537 | 1606 | 20 | 0324 | 0353 | 1608 | 1637 | 20 | 0233 | 0304 | 1641 | 1711 |
| 21 | 0437 | 0508 | 1500 | 1532 | 21 | 0406 | 0436 | 1538 | 1607 | 21 | 0323 | 0352 | 1609 | 1638 | 21 | 0231 | 0302 | 1642 | 1713 |
| 22 | 0436 | 0508 | 1501 | 1533 | 22 | 0405 | 0434 | 1539 | 1608 | 22 | 0321 | 0350 | 1610 | 1639 | 22 | 0230 | 0301 | 1643 | 1714 |
| 23 | 0436 | 0507 | 1503 | 1534 | 23 | 0404 | 0433 | 1540 | 1610 | 23 | 0319 | 0349 | 1611 | 1640 | 23 | 0228 | 0259 | 1644 | 1715 |
| 24 | 0435 | 0506 | 1504 | 1535 | 24 | 0402 | 0431 | 1541 | 1611 | 24 | 0318 | 0347 | 1612 | 1641 | 24 | 0227 | 0258 | 1645 | 1716 |
| 25 | 0435 | 0506 | 1505 | 1536 | 25 | 0401 | 0430 | 1542 | 1612 | 25 | 0316 | 0345 | 1613 | 1642 | 25 | 0225 | 0256 | 1646 | 1717 |
| 26 | 0434 | 0505 | 1506 | 1537 | 26 | 0359 | 0429 | 1544 | 1613 | 26 | 0314 | 0344 | 1614 | 1644 | 26 | 0224 | 0255 | 1647 | 1718 |
| 27 | 0433 | 0504 | 1508 | 1539 | 27 | 0358 | 0427 | 1545 | 1614 | 27 | 0313 | 0342 | 1615 | 1645 | 27 | 0222 | 0254 | 1648 | 1720 |
| 28 | 0432 | 0503 | 1509 | 1540 | 28 | 0356 | 0426 | 1546 | 1615 | 28 | 0311 | 0340 | 1616 | 1646 | 28 | 0221 | 0252 | 1649 | 1721 |
| 29 | 0432 | 0502 | 1510 | 1541 |  |  |  |  |  | 29 | 0309 | 0339 | 1617 | 1647 | 29 | 0219 | 0251 | 1650 | 1722 |
| 30 | 0431 | 0502 | 1511 | 1542 |  |  |  |  |  | 30 | 0308 | 0337 | 1619 | 1648 | 30 | 0218 | 0250 | 1651 | 1723 |
| 31 | 0430 | 0501 | 1512 | 1543 |  |  |  |  |  | 31 | 0306 | 0335 | 1620 | 1649 |  |  |  |  |  |

| MAY - 2026 |  |  |  |  | JUNE - 2026 |  |  |  |  |  | JULY - 2026 |  |  |  |  |  | AUGUST - 2026 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DAY | TWIL FROM | SR | SS | TWIL TO | DAY | TWIL FROM | SR | SS | TWIL TO | DAY |  | TWIL FROM | SR | SS | TWIL TO | DAY |  | TWIL FROM | SR | SS | TWIL TO |
| 1 | 0217 | 0248 | 1652 | 1724 | 1 | 0145 | 0220 | 1722 | 1757 | 1 |  | 0146 | 0222 | 1732 | 1808 | 1 |  | 0213 | 0246 | 1712 | 1745 |
| 2 | 0215 | 0247 | 1653 | 1725 | 2 | 0145 | 0220 | 1722 | 1757 | 2 |  | 0146 | 0222 | 1732 | 1807 | 2 |  | 0214 | 0247 | 1711 | 1744 |
| 3 | 0214 | 0246 | 1654 | 1726 | 3 | 0144 | 0219 | 1723 | 1758 | 3 |  | 0147 | 0223 | 1732 | 1807 | 3 |  | 0215 | 0248 | 1710 | 1742 |
| 4 | 0213 | 0245 | 1655 | 1728 | 4 | 0144 | 0219 | 1724 | 1759 | 4 |  | 0148 | 0223 | 1731 | 1807 | 4 |  | 0216 | 0249 | 1709 | 1741 |
| 5 | 0211 | 0243 | 1657 | 1729 | 5 | 0143 | 0219 | 1724 | 1800 | 5 |  | 0148 | 0224 | 1731 | 1807 | 5 |  | 0217 | 0250 | 1708 | 1740 |
| 6 | 0210 | 0242 | 1658 | 1730 | 6 | 0143 | 0218 | 1725 | 1800 | 6 |  | 0149 | 0224 | 1731 | 1806 | 6 |  | 0218 | 0251 | 1706 | 1739 |
| 7 | 0209 | 0241 | 1659 | 1731 | 7 | 0143 | 0218 | 1726 | 1801 | 7 |  | 0150 | 0225 | 1730 | 1806 | 7 |  | 0219 | 0252 | 1705 | 1737 |
| 8 | 0207 | 0240 | 1700 | 1732 | 8 | 0142 | 0218 | 1726 | 1802 | 8 |  | 0150 | 0226 | 1730 | 1805 | 8 |  | 0220 | 0253 | 1704 | 1736 |
| 9 | 0206 | 0239 | 1701 | 1733 | 9 | 0142 | 0218 | 1727 | 1802 | 9 |  | 0151 | 0226 | 1730 | 1805 | 9 |  | 0222 | 0254 | 1703 | 1735 |
| 10 | 0205 | 0238 | 1702 | 1734 | 10 | 0142 | 0218 | 1727 | 1803 | 10 |  | 0152 | 0227 | 1729 | 1804 | 10 |  | 0223 | 0255 | 1701 | 1733 |
| 11 | 0204 | 0237 | 1703 | 1736 | 11 | 0142 | 0217 | 1728 | 1804 | 11 |  | 0153 | 0228 | 1729 | 1804 | 11 |  | 0224 | 0256 | 1700 | 1732 |
| 12 | 0203 | 0235 | 1704 | 1737 | 12 | 0142 | 0217 | 1728 | 1804 | 12 |  | 0153 | 0228 | 1728 | 1803 | 12 |  | 0225 | 0257 | 1659 | 1730 |
| 13 | 0201 | 0234 | 1705 | 1738 | 13 | 0142 | 0217 | 1729 | 1805 | 13 |  | 0154 | 0229 | 1728 | 1803 | 13 |  | 0226 | 0258 | 1657 | 1729 |
| 14 | 0200 | 0233 | 1706 | 1739 | 14 | 0141 | 0217 | 1729 | 1805 | 14 |  | 0155 | 0230 | 1727 | 1802 | 14 |  | 0227 | 0259 | 1656 | 1727 |
| 15 | 0159 | 0232 | 1707 | 1740 | 15 | 0141 | 0217 | 1730 | 1806 | 15 |  | 0156 | 0231 | 1727 | 1801 | 15 |  | 0228 | 0300 | 1655 | 1726 |
| 16 | 0158 | 0231 | 1708 | 1741 | 16 | 0141 | 0217 | 1730 | 1806 | 16 |  | 0157 | 0231 | 1726 | 1801 | 16 |  | 0229 | 0301 | 1653 | 1724 |
| 17 | 0157 | 0231 | 1709 | 1742 | 17 | 0141 | 0217 | 1730 | 1806 | 17 |  | 0158 | 0232 | 1725 | 1800 | 17 |  | 0230 | 0302 | 1652 | 1723 |
| 18 | 0156 | 0230 | 1710 | 1743 | 18 | 0142 | 0217 | 1731 | 1807 | 18 |  | 0159 | 0233 | 1725 | 1759 | 18 |  | 0231 | 0303 | 1650 | 1721 |
| 19 | 0155 | 0229 | 1711 | 1744 | 19 | 0142 | 0218 | 1731 | 1807 | 19 |  | 0200 | 0234 | 1724 | 1758 | 19 |  | 0232 | 0304 | 1649 | 1720 |
| 20 | 0154 | 0228 | 1711 | 1745 | 20 | 0142 | 0218 | 1731 | 1807 | 20 |  | 0201 | 0235 | 1723 | 1757 | 20 |  | 0234 | 0305 | 1647 | 1718 |
| 21 | 0153 | 0227 | 1712 | 1746 | 21 | 0142 | 0218 | 1731 | 1807 | 21 |  | 0202 | 0236 | 1723 | 1757 | 21 |  | 0235 | 0306 | 1646 | 1717 |
| 22 | 0152 | 0226 | 1713 | 1747 | 22 | 0142 | 0218 | 1732 | 1808 | 22 |  | 0202 | 0237 | 1722 | 1756 | 22 |  | 0236 | 0307 | 1644 | 1715 |
| 23 | 0151 | 0226 | 1714 | 1748 | 23 | 0142 | 0218 | 1732 | 1808 | 23 |  | 0203 | 0237 | 1721 | 1755 | 23 |  | 0237 | 0308 | 1643 | 1713 |
| 24 | 0151 | 0225 | 1715 | 1749 | 24 | 0143 | 0219 | 1732 | 1808 | 24 |  | 0204 | 0238 | 1720 | 1754 | 24 |  | 0238 | 0309 | 1641 | 1712 |
| 25 | 0150 | 0224 | 1716 | 1750 | 25 | 0143 | 0219 | 1732 | 1808 | 25 |  | 0205 | 0239 | 1719 | 1753 | 25 |  | 0239 | 0310 | 1640 | 1710 |
| 26 | 0149 | 0224 | 1717 | 1751 | 26 | 0143 | 0219 | 1732 | 1808 | 26 |  | 0207 | 0240 | 1718 | 1752 | 26 |  | 0240 | 0311 | 1638 | 1709 |
| 27 | 0148 | 0223 | 1718 | 1752 | 27 | 0144 | 0220 | 1732 | 1808 | 27 |  | 0208 | 0241 | 1717 | 1751 | 27 |  | 0241 | 0312 | 1637 | 1707 |
| 28 | 0148 | 0222 | 1718 | 1753 | 28 | 0144 | 0220 | 1732 | 1808 | 28 |  | 0209 | 0242 | 1716 | 1750 | 28 |  | 0242 | 0312 | 1635 | 1705 |
| 29 | 0147 | 0222 | 1719 | 1754 | 29 | 0145 | 0221 | 1732 | 1808 | 29 |  | 0210 | 0243 | 1715 | 1748 | 29 |  | 0243 | 0313 | 1634 | 1704 |
| 30 | 0146 | 0221 | 1720 | 1755 | 30 | 0145 | 0221 | 1732 | 1808 | 30 |  | 0211 | 0244 | 1714 | 1747 | 30 |  | 0244 | 0314 | 1632 | 1702 |
| 31 | 0146 | 0221 | 1721 | 1756 |  |  |  |  |  | 31 |  | 0212 | 0245 | 1713 | 1746 | 31 |  | 0245 | 0315 | 1630 | 1700 |

| SEPTEMBER - 2026 |  |  |  |  | OCTOBER - 2026 |  |  |  |  |  | NOVEMBER - 2026 |  |  |  |  |  | DECEMBER - 2026 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DAY | TWIL FROM | SR | SS | TWIL TO | DAY | TWIL FROM | SR | SS | TWIL TO | DAY |  | TWIL FROM | SR | SS | TWIL TO | DAY |  | TWIL FROM | SR | SS | TWIL TO |
| 1 | 0246 | 0316 | 1629 | 1659 | 1 | 0317 | 0346 | 1538 | 1607 | 1 |  | 0350 | 0420 | 1453 | 1523 | 1 |  | 0422 | 0454 | 1430 | 1502 |
| 2 | 0247 | 0317 | 1627 | 1657 | 2 | 0318 | 0347 | 1537 | 1606 | 2 |  | 0351 | 0421 | 1451 | 1522 | 2 |  | 0423 | 0455 | 1429 | 1502 |
| 3 | 0249 | 0318 | 1625 | 1655 | 3 | 0319 | 0348 | 1535 | 1604 | 3 |  | 0352 | 0422 | 1450 | 1521 | 3 |  | 0424 | 0456 | 1429 | 1501 |
| 4 | 0250 | 0319 | 1624 | 1654 | 4 | 0320 | 0349 | 1534 | 1603 | 4 |  | 0353 | 0423 | 1449 | 1519 | 4 |  | 0425 | 0457 | 1429 | 1501 |
| 5 | 0251 | 0320 | 1622 | 1652 | 5 | 0321 | 0350 | 1532 | 1601 | 5 |  | 0354 | 0424 | 1448 | 1518 | 5 |  | 0425 | 0458 | 1429 | 1501 |
| 6 | 0252 | 0321 | 1621 | 1650 | 6 | 0322 | 0351 | 1530 | 1559 | 6 |  | 0355 | 0425 | 1447 | 1517 | 6 |  | 0426 | 0459 | 1429 | 1501 |
| 7 | 0253 | 0322 | 1619 | 1648 | 7 | 0323 | 0352 | 1529 | 1558 | 7 |  | 0356 | 0427 | 1446 | 1516 | 7 |  | 0427 | 0500 | 1429 | 1501 |
| 8 | 0254 | 0323 | 1617 | 1647 | 8 | 0324 | 0353 | 1527 | 1556 | 8 |  | 0357 | 0428 | 1445 | 1515 | 8 |  | 0428 | 0501 | 1429 | 1501 |
| 9 | 0255 | 0324 | 1616 | 1645 | 9 | 0325 | 0354 | 1525 | 1555 | 9 |  | 0358 | 0429 | 1444 | 1514 | 9 |  | 0429 | 0501 | 1429 | 1501 |
| 10 | 0256 | 0325 | 1614 | 1643 | 10 | 0326 | 0355 | 1524 | 1553 | 10 |  | 0359 | 0430 | 1443 | 1514 | 10 |  | 0430 | 0502 | 1429 | 1501 |
| 11 | 0257 | 0326 | 1612 | 1642 | 11 | 0327 | 0356 | 1522 | 1551 | 11 |  | 0401 | 0431 | 1442 | 1513 | 11 |  | 0430 | 0503 | 1429 | 1502 |
| 12 | 0258 | 0327 | 1611 | 1640 | 12 | 0328 | 0357 | 1521 | 1550 | 12 |  | 0402 | 0433 | 1441 | 1512 | 12 |  | 0431 | 0504 | 1429 | 1502 |
| 13 | 0259 | 0328 | 1609 | 1638 | 13 | 0329 | 0358 | 1519 | 1548 | 13 |  | 0403 | 0434 | 1440 | 1511 | 13 |  | 0432 | 0505 | 1429 | 1502 |
| 14 | 0300 | 0329 | 1607 | 1636 | 14 | 0330 | 0400 | 1518 | 1547 | 14 |  | 0404 | 0435 | 1439 | 1510 | 14 |  | 0433 | 0505 | 1430 | 1502 |
| 15 | 0301 | 0330 | 1605 | 1635 | 15 | 0331 | 0401 | 1516 | 1545 | 15 |  | 0405 | 0436 | 1438 | 1509 | 15 |  | 0433 | 0506 | 1430 | 1503 |
| 16 | 0302 | 0331 | 1604 | 1633 | 16 | 0332 | 0402 | 1515 | 1544 | 16 |  | 0406 | 0437 | 1438 | 1509 | 16 |  | 0434 | 0507 | 1430 | 1503 |
| 17 | 0303 | 0332 | 1602 | 1631 | 17 | 0333 | 0403 | 1513 | 1542 | 17 |  | 0407 | 0438 | 1437 | 1508 | 17 |  | 0435 | 0507 | 1430 | 1503 |
| 18 | 0304 | 0333 | 1600 | 1629 | 18 | 0334 | 0404 | 1512 | 1541 | 18 |  | 0408 | 0440 | 1436 | 1507 | 18 |  | 0435 | 0508 | 1431 | 1504 |
| 19 | 0305 | 0334 | 1559 | 1628 | 19 | 0336 | 0405 | 1510 | 1540 | 19 |  | 0409 | 0441 | 1435 | 1507 | 19 |  | 0436 | 0509 | 1431 | 1504 |
| 20 | 0306 | 0335 | 1557 | 1626 | 20 | 0337 | 0406 | 1509 | 1538 | 20 |  | 0410 | 0442 | 1435 | 1506 | 20 |  | 0436 | 0509 | 1432 | 1504 |
| 21 | 0307 | 0336 | 1555 | 1624 | 21 | 0338 | 0407 | 1507 | 1537 | 21 |  | 0411 | 0443 | 1434 | 1506 | 21 |  | 0437 | 0510 | 1432 | 1505 |
| 22 | 0308 | 0337 | 1554 | 1623 | 22 | 0339 | 0408 | 1506 | 1535 | 22 |  | 0413 | 0444 | 1433 | 1505 | 22 |  | 0437 | 0510 | 1433 | 1505 |
| 23 | 0309 | 0338 | 1552 | 1621 | 23 | 0340 | 0409 | 1504 | 1534 | 23 |  | 0414 | 0445 | 1433 | 1505 | 23 |  | 0438 | 0511 | 1433 | 1506 |
| 24 | 0310 | 0339 | 1550 | 1619 | 24 | 0341 | 0411 | 1503 | 1533 | 24 |  | 0415 | 0446 | 1432 | 1504 | 24 |  | 0438 | 0511 | 1434 | 1506 |
| 25 | 0311 | 0340 | 1549 | 1618 | 25 | 0342 | 0412 | 1502 | 1531 | 25 |  | 0416 | 0447 | 1432 | 1504 | 25 |  | 0439 | 0512 | 1434 | 1507 |
| 26 | 0312 | 0341 | 1547 | 1616 | 26 | 0343 | 0413 | 1500 | 1530 | 26 |  | 0417 | 0449 | 1431 | 1503 | 26 |  | 0439 | 0512 | 1435 | 1508 |
| 27 | 0313 | 0342 | 1545 | 1614 | 27 | 0344 | 0414 | 1459 | 1529 | 27 |  | 0418 | 0450 | 1431 | 1503 | 27 |  | 0440 | 0512 | 1436 | 1508 |
| 28 | 0314 | 0343 | 1543 | 1612 | 28 | 0345 | 0415 | 1458 | 1528 | 28 |  | 0419 | 0451 | 1431 | 1503 | 28 |  | 0440 | 0513 | 1436 | 1509 |
| 29 | 0315 | 0344 | 1542 | 1611 | 29 | 0346 | 0416 | 1456 | 1526 | 29 |  | 0420 | 0452 | 1430 | 1502 | 29 |  | 0440 | 0513 | 1437 | 1510 |
| 30 | 0316 | 0345 | 1540 | 1609 | 30 | 0347 | 0417 | 1455 | 1525 | 30 |  | 0421 | 0453 | 1430 | 1502 | 30 |  | 0440 | 0513 | 1438 | 1510 |
|  |  |  |  |  | 31 | 0348 | 0419 | 1454 | 1524 |  |  |  |  |  |  | 31 |  | 0441 | 0513 | 1439 | 1511 |

#### Year 2027 (UTC)

| JANUARY - 2027 |  |  |  |  | FEBRUARY - 2027 |  |  |  |  | MARCH - 2027 |  |  |  |  | APRIL - 2027 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DAY | TWIL FROM | SR | SS | TWIL TO | DAY | TWIL FROM | SR | SS | TWIL TO | DAY | TWIL FROM | SR | SS | TWIL TO | DAY | TWIL FROM | SR | SS | TWIL TO |
| 1 | 0441 | 0513 | 1439 | 1512 | 1 | 0429 | 0500 | 1513 | 1544 | 1 | 0355 | 0424 | 1547 | 1616 | 1 | 0305 | 0334 | 1620 | 1650 |
| 2 | 0441 | 0513 | 1440 | 1513 | 2 | 0428 | 0459 | 1515 | 1545 | 2 | 0354 | 0423 | 1548 | 1617 | 2 | 0303 | 0332 | 1621 | 1651 |
| 3 | 0441 | 0514 | 1441 | 1514 | 3 | 0427 | 0458 | 1516 | 1546 | 3 | 0352 | 0421 | 1549 | 1618 | 3 | 0301 | 0331 | 1622 | 1652 |
| 4 | 0441 | 0514 | 1442 | 1514 | 4 | 0427 | 0457 | 1517 | 1548 | 4 | 0351 | 0420 | 1550 | 1619 | 4 | 0300 | 0329 | 1624 | 1653 |
| 5 | 0441 | 0514 | 1443 | 1515 | 5 | 0426 | 0456 | 1518 | 1549 | 5 | 0349 | 0418 | 1551 | 1620 | 5 | 0258 | 0327 | 1625 | 1654 |
| 6 | 0441 | 0514 | 1444 | 1516 | 6 | 0425 | 0455 | 1519 | 1550 | 6 | 0348 | 0417 | 1552 | 1622 | 6 | 0256 | 0326 | 1626 | 1655 |
| 7 | 0441 | 0513 | 1445 | 1517 | 7 | 0424 | 0454 | 1521 | 1551 | 7 | 0346 | 0415 | 1554 | 1623 | 7 | 0255 | 0324 | 1627 | 1656 |
| 8 | 0441 | 0513 | 1446 | 1518 | 8 | 0422 | 0453 | 1522 | 1552 | 8 | 0344 | 0413 | 1555 | 1624 | 8 | 0253 | 0323 | 1628 | 1658 |
| 9 | 0441 | 0513 | 1447 | 1519 | 9 | 0421 | 0452 | 1523 | 1553 | 9 | 0343 | 0412 | 1556 | 1625 | 9 | 0251 | 0321 | 1629 | 1659 |
| 10 | 0441 | 0513 | 1448 | 1520 | 10 | 0420 | 0450 | 1524 | 1554 | 10 | 0341 | 0410 | 1557 | 1626 | 10 | 0250 | 0319 | 1630 | 1700 |
| 11 | 0441 | 0513 | 1449 | 1521 | 11 | 0419 | 0449 | 1526 | 1556 | 11 | 0340 | 0409 | 1558 | 1627 | 11 | 0248 | 0318 | 1631 | 1701 |
| 12 | 0440 | 0513 | 1450 | 1522 | 12 | 0418 | 0448 | 1527 | 1557 | 12 | 0338 | 0407 | 1559 | 1628 | 12 | 0246 | 0316 | 1632 | 1702 |
| 13 | 0440 | 0512 | 1451 | 1523 | 13 | 0417 | 0447 | 1528 | 1558 | 13 | 0336 | 0405 | 1600 | 1629 | 13 | 0245 | 0315 | 1633 | 1703 |
| 14 | 0440 | 0512 | 1452 | 1524 | 14 | 0416 | 0445 | 1529 | 1559 | 14 | 0335 | 0404 | 1601 | 1630 | 14 | 0243 | 0313 | 1634 | 1704 |
| 15 | 0440 | 0512 | 1453 | 1525 | 15 | 0414 | 0444 | 1530 | 1600 | 15 | 0333 | 0402 | 1602 | 1631 | 15 | 0241 | 0312 | 1635 | 1705 |
| 16 | 0439 | 0511 | 1454 | 1526 | 16 | 0413 | 0443 | 1532 | 1601 | 16 | 0332 | 0400 | 1603 | 1632 | 16 | 0240 | 0310 | 1636 | 1707 |
| 17 | 0439 | 0511 | 1455 | 1527 | 17 | 0412 | 0442 | 1533 | 1603 | 17 | 0330 | 0359 | 1604 | 1633 | 17 | 0238 | 0309 | 1637 | 1708 |
| 18 | 0439 | 0510 | 1457 | 1528 | 18 | 0411 | 0440 | 1534 | 1604 | 18 | 0328 | 0357 | 1606 | 1635 | 18 | 0237 | 0307 | 1638 | 1709 |
| 19 | 0438 | 0510 | 1458 | 1529 | 19 | 0409 | 0439 | 1535 | 1605 | 19 | 0327 | 0356 | 1607 | 1636 | 19 | 0235 | 0306 | 1639 | 1710 |
| 20 | 0438 | 0509 | 1459 | 1530 | 20 | 0408 | 0437 | 1536 | 1606 | 20 | 0325 | 0354 | 1608 | 1637 | 20 | 0233 | 0304 | 1640 | 1711 |
| 21 | 0437 | 0509 | 1500 | 1532 | 21 | 0407 | 0436 | 1538 | 1607 | 21 | 0323 | 0352 | 1609 | 1638 | 21 | 0232 | 0303 | 1641 | 1712 |
| 22 | 0437 | 0508 | 1501 | 1533 | 22 | 0405 | 0435 | 1539 | 1608 | 22 | 0322 | 0351 | 1610 | 1639 | 22 | 0230 | 0301 | 1643 | 1713 |
| 23 | 0436 | 0507 | 1502 | 1534 | 23 | 0404 | 0433 | 1540 | 1609 | 23 | 0320 | 0349 | 1611 | 1640 | 23 | 0229 | 0300 | 1644 | 1715 |
| 24 | 0435 | 0507 | 1504 | 1535 | 24 | 0402 | 0432 | 1541 | 1610 | 24 | 0318 | 0347 | 1612 | 1641 | 24 | 0227 | 0258 | 1645 | 1716 |
| 25 | 0435 | 0506 | 1505 | 1536 | 25 | 0401 | 0430 | 1542 | 1612 | 25 | 0316 | 0346 | 1613 | 1642 | 25 | 0226 | 0257 | 1646 | 1717 |
| 26 | 0434 | 0505 | 1506 | 1537 | 26 | 0400 | 0429 | 1543 | 1613 | 26 | 0315 | 0344 | 1614 | 1643 | 26 | 0224 | 0255 | 1647 | 1718 |
| 27 | 0433 | 0504 | 1507 | 1538 | 27 | 0358 | 0427 | 1545 | 1614 | 27 | 0313 | 0342 | 1615 | 1644 | 27 | 0223 | 0254 | 1648 | 1719 |
| 28 | 0433 | 0504 | 1508 | 1539 | 28 | 0357 | 0426 | 1546 | 1615 | 28 | 0311 | 0341 | 1616 | 1645 | 28 | 0221 | 0253 | 1649 | 1720 |
| 29 | 0432 | 0503 | 1510 | 1541 |  |  |  |  |  | 29 | 0310 | 0339 | 1617 | 1647 | 29 | 0220 | 0251 | 1650 | 1722 |
| 30 | 0431 | 0502 | 1511 | 1542 |  |  |  |  |  | 30 | 0308 | 0337 | 1618 | 1648 | 30 | 0218 | 0250 | 1651 | 1723 |
| 31 | 0430 | 0501 | 1512 | 1543 |  |  |  |  |  | 31 | 0306 | 0336 | 1619 | 1649 |  |  |  |  |  |

| MAY - 2027 |  |  |  |  | JUNE - 2027 |  |  |  |  | JULY - 2027 |  |  |  |  | AUGUST - 2027 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DAY | TWIL FROM | SR | SS | TWIL TO | DAY | TWIL FROM | SR | SS | TWIL TO | DAY | TWIL FROM | SR | SS | TWIL TO | DAY | TWIL FROM | SR | SS | TWIL TO |
| 1 | 0217 | 0249 | 1652 | 1724 | 1 | 0145 | 0220 | 1721 | 1756 | 1 | 0146 | 0221 | 1732 | 1808 | 1 | 0213 | 0246 | 1712 | 1745 |
| 2 | 0216 | 0247 | 1653 | 1725 | 2 | 0145 | 0220 | 1722 | 1757 | 2 | 0146 | 0222 | 1732 | 1807 | 2 | 0214 | 0247 | 1711 | 1744 |
| 3 | 0214 | 0246 | 1654 | 1726 | 3 | 0144 | 0220 | 1723 | 1758 | 3 | 0147 | 0222 | 1732 | 1807 | 3 | 0215 | 0248 | 1710 | 1743 |
| 4 | 0213 | 0245 | 1655 | 1727 | 4 | 0144 | 0219 | 1724 | 1759 | 4 | 0147 | 0223 | 1731 | 1807 | 4 | 0216 | 0248 | 1709 | 1742 |
| 5 | 0212 | 0244 | 1656 | 1729 | 5 | 0144 | 0219 | 1724 | 1800 | 5 | 0148 | 0224 | 1731 | 1807 | 5 | 0217 | 0249 | 1708 | 1740 |
| 6 | 0210 | 0242 | 1657 | 1730 | 6 | 0143 | 0219 | 1725 | 1800 | 6 | 0149 | 0224 | 1731 | 1806 | 6 | 0218 | 0250 | 1707 | 1739 |
| 7 | 0209 | 0241 | 1658 | 1731 | 7 | 0143 | 0218 | 1725 | 1801 | 7 | 0149 | 0225 | 1731 | 1806 | 7 | 0219 | 0251 | 1705 | 1738 |
| 8 | 0208 | 0240 | 1659 | 1732 | 8 | 0142 | 0218 | 1726 | 1802 | 8 | 0150 | 0225 | 1730 | 1805 | 8 | 0220 | 0252 | 1704 | 1736 |
| 9 | 0206 | 0239 | 1700 | 1733 | 9 | 0142 | 0218 | 1727 | 1802 | 9 | 0151 | 0226 | 1730 | 1805 | 9 | 0221 | 0253 | 1703 | 1735 |
| 10 | 0205 | 0238 | 1701 | 1734 | 10 | 0142 | 0218 | 1727 | 1803 | 10 | 0152 | 0227 | 1729 | 1805 | 10 | 0222 | 0254 | 1702 | 1733 |
| 11 | 0204 | 0237 | 1702 | 1735 | 11 | 0142 | 0217 | 1728 | 1803 | 11 | 0152 | 0227 | 1729 | 1804 | 11 | 0223 | 0255 | 1700 | 1732 |
| 12 | 0203 | 0236 | 1703 | 1736 | 12 | 0142 | 0217 | 1728 | 1804 | 12 | 0153 | 0228 | 1728 | 1803 | 12 | 0225 | 0256 | 1659 | 1731 |
| 13 | 0202 | 0235 | 1704 | 1738 | 13 | 0142 | 0217 | 1729 | 1805 | 13 | 0154 | 0229 | 1728 | 1803 | 13 | 0226 | 0257 | 1658 | 1729 |
| 14 | 0201 | 0234 | 1705 | 1739 | 14 | 0141 | 0217 | 1729 | 1805 | 14 | 0155 | 0230 | 1727 | 1802 | 14 | 0227 | 0258 | 1656 | 1728 |
| 15 | 0159 | 0233 | 1706 | 1740 | 15 | 0141 | 0217 | 1730 | 1805 | 15 | 0156 | 0230 | 1727 | 1802 | 15 | 0228 | 0259 | 1655 | 1726 |
| 16 | 0158 | 0232 | 1707 | 1741 | 16 | 0141 | 0217 | 1730 | 1806 | 16 | 0157 | 0231 | 1726 | 1801 | 16 | 0229 | 0300 | 1653 | 1725 |
| 17 | 0157 | 0231 | 1708 | 1742 | 17 | 0141 | 0217 | 1730 | 1806 | 17 | 0158 | 0232 | 1726 | 1800 | 17 | 0230 | 0301 | 1652 | 1723 |
| 18 | 0156 | 0230 | 1709 | 1743 | 18 | 0142 | 0217 | 1731 | 1807 | 18 | 0158 | 0233 | 1725 | 1759 | 18 | 0231 | 0302 | 1651 | 1722 |
| 19 | 0155 | 0229 | 1710 | 1744 | 19 | 0142 | 0218 | 1731 | 1807 | 19 | 0159 | 0234 | 1724 | 1758 | 19 | 0232 | 0303 | 1649 | 1720 |
| 20 | 0154 | 0228 | 1711 | 1745 | 20 | 0142 | 0218 | 1731 | 1807 | 20 | 0200 | 0235 | 1723 | 1758 | 20 | 0233 | 0304 | 1648 | 1719 |
| 21 | 0153 | 0227 | 1712 | 1746 | 21 | 0142 | 0218 | 1731 | 1807 | 21 | 0201 | 0235 | 1723 | 1757 | 21 | 0234 | 0305 | 1646 | 1717 |
| 22 | 0153 | 0227 | 1713 | 1747 | 22 | 0142 | 0218 | 1732 | 1808 | 22 | 0202 | 0236 | 1722 | 1756 | 22 | 0235 | 0306 | 1645 | 1715 |
| 23 | 0152 | 0226 | 1714 | 1748 | 23 | 0142 | 0218 | 1732 | 1808 | 23 | 0203 | 0237 | 1721 | 1755 | 23 | 0237 | 0307 | 1643 | 1714 |
| 24 | 0151 | 0225 | 1715 | 1749 | 24 | 0143 | 0219 | 1732 | 1808 | 24 | 0204 | 0238 | 1720 | 1754 | 24 | 0238 | 0308 | 1642 | 1712 |
| 25 | 0150 | 0224 | 1716 | 1750 | 25 | 0143 | 0219 | 1732 | 1808 | 25 | 0205 | 0239 | 1719 | 1753 | 25 | 0239 | 0309 | 1640 | 1711 |
| 26 | 0149 | 0224 | 1717 | 1751 | 26 | 0143 | 0219 | 1732 | 1808 | 26 | 0206 | 0240 | 1718 | 1752 | 26 | 0240 | 0310 | 1639 | 1709 |
| 27 | 0149 | 0223 | 1717 | 1752 | 27 | 0144 | 0220 | 1732 | 1808 | 27 | 0207 | 0241 | 1717 | 1751 | 27 | 0241 | 0311 | 1637 | 1707 |
| 28 | 0148 | 0222 | 1718 | 1753 | 28 | 0144 | 0220 | 1732 | 1808 | 28 | 0208 | 0242 | 1717 | 1750 | 28 | 0242 | 0312 | 1636 | 1706 |
| 29 | 0147 | 0222 | 1719 | 1754 | 29 | 0145 | 0220 | 1732 | 1808 | 29 | 0209 | 0243 | 1716 | 1749 | 29 | 0243 | 0313 | 1634 | 1704 |
| 30 | 0147 | 0221 | 1720 | 1755 | 30 | 0145 | 0221 | 1732 | 1808 | 30 | 0210 | 0244 | 1714 | 1748 | 30 | 0244 | 0314 | 1632 | 1702 |
| 31 | 0146 | 0221 | 1721 | 1756 |  |  |  |  |  | 31 | 0212 | 0245 | 1713 | 1746 | 31 | 0245 | 0315 | 1631 | 1701 |

| SEPTEMBER - 2027 |  |  |  |  | OCTOBER - 2027 |  |  |  |  | NOVEMBER - 2027 |  |  |  |  | DECEMBER - 2027 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DAY | TWIL FROM | SR | SS | TWIL TO | DAY | TWIL FROM | SR | SS | TWIL TO | DAY | TWIL FROM | SR | SS | TWIL TO | DAY | TWIL FROM | SR | SS | TWIL TO |
| 1 | 0246 | 0316 | 1629 | 1659 | 1 | 0317 | 0346 | 1539 | 1608 | 1 | 0349 | 0419 | 1453 | 1523 | 1 | 0421 | 0454 | 1430 | 1502 |
| 2 | 0247 | 0317 | 1627 | 1657 | 2 | 0318 | 0347 | 1537 | 1606 | 2 | 0350 | 0421 | 1452 | 1522 | 2 | 0422 | 0455 | 1430 | 1502 |
| 3 | 0248 | 0318 | 1626 | 1656 | 3 | 0319 | 0348 | 1536 | 1605 | 3 | 0351 | 0422 | 1451 | 1521 | 3 | 0423 | 0456 | 1429 | 1502 |
| 4 | 0249 | 0319 | 1624 | 1654 | 4 | 0320 | 0349 | 1534 | 1603 | 4 | 0353 | 0423 | 1449 | 1520 | 4 | 0424 | 0457 | 1429 | 1501 |
| 5 | 0250 | 0320 | 1623 | 1652 | 5 | 0321 | 0350 | 1532 | 1601 | 5 | 0354 | 0424 | 1448 | 1519 | 5 | 0425 | 0458 | 1429 | 1501 |
| 6 | 0251 | 0321 | 1621 | 1651 | 6 | 0322 | 0351 | 1531 | 1600 | 6 | 0355 | 0425 | 1447 | 1518 | 6 | 0426 | 0458 | 1429 | 1501 |
| 7 | 0252 | 0322 | 1619 | 1649 | 7 | 0323 | 0352 | 1529 | 1558 | 7 | 0356 | 0426 | 1446 | 1517 | 7 | 0427 | 0459 | 1429 | 1501 |
| 8 | 0253 | 0323 | 1618 | 1647 | 8 | 0324 | 0353 | 1527 | 1556 | 8 | 0357 | 0428 | 1445 | 1516 | 8 | 0428 | 0500 | 1429 | 1501 |
| 9 | 0255 | 0324 | 1616 | 1645 | 9 | 0325 | 0354 | 1526 | 1555 | 9 | 0358 | 0429 | 1444 | 1515 | 9 | 0429 | 0501 | 1429 | 1501 |
| 10 | 0256 | 0325 | 1614 | 1644 | 10 | 0326 | 0355 | 1524 | 1553 | 10 | 0359 | 0430 | 1443 | 1514 | 10 | 0429 | 0502 | 1429 | 1501 |
| 11 | 0257 | 0326 | 1613 | 1642 | 11 | 0327 | 0356 | 1523 | 1552 | 11 | 0400 | 0431 | 1442 | 1513 | 11 | 0430 | 0503 | 1429 | 1502 |
| 12 | 0258 | 0327 | 1611 | 1640 | 12 | 0328 | 0357 | 1521 | 1550 | 12 | 0401 | 0432 | 1441 | 1512 | 12 | 0431 | 0504 | 1429 | 1502 |
| 13 | 0259 | 0328 | 1609 | 1639 | 13 | 0329 | 0358 | 1520 | 1549 | 13 | 0403 | 0433 | 1440 | 1511 | 13 | 0432 | 0504 | 1429 | 1502 |
| 14 | 0300 | 0329 | 1608 | 1637 | 14 | 0330 | 0359 | 1518 | 1547 | 14 | 0404 | 0435 | 1439 | 1510 | 14 | 0433 | 0505 | 1430 | 1502 |
| 15 | 0301 | 0330 | 1606 | 1635 | 15 | 0331 | 0400 | 1516 | 1546 | 15 | 0405 | 0436 | 1439 | 1510 | 15 | 0433 | 0506 | 1430 | 1502 |
| 16 | 0302 | 0331 | 1604 | 1633 | 16 | 0332 | 0401 | 1515 | 1544 | 16 | 0406 | 0437 | 1438 | 1509 | 16 | 0434 | 0507 | 1430 | 1503 |
| 17 | 0303 | 0332 | 1602 | 1632 | 17 | 0333 | 0403 | 1513 | 1543 | 17 | 0407 | 0438 | 1437 | 1508 | 17 | 0435 | 0507 | 1430 | 1503 |
| 18 | 0304 | 0333 | 1601 | 1630 | 18 | 0334 | 0404 | 1512 | 1541 | 18 | 0408 | 0439 | 1436 | 1508 | 18 | 0435 | 0508 | 1431 | 1503 |
| 19 | 0305 | 0334 | 1559 | 1628 | 19 | 0335 | 0405 | 1510 | 1540 | 19 | 0409 | 0440 | 1436 | 1507 | 19 | 0436 | 0509 | 1431 | 1504 |
| 20 | 0306 | 0335 | 1557 | 1626 | 20 | 0336 | 0406 | 1509 | 1538 | 20 | 0410 | 0442 | 1435 | 1506 | 20 | 0436 | 0509 | 1432 | 1504 |
| 21 | 0307 | 0336 | 1556 | 1625 | 21 | 0337 | 0407 | 1508 | 1537 | 21 | 0411 | 0443 | 1434 | 1506 | 21 | 0437 | 0510 | 1432 | 1505 |
| 22 | 0308 | 0337 | 1554 | 1623 | 22 | 0338 | 0408 | 1506 | 1536 | 22 | 0412 | 0444 | 1434 | 1505 | 22 | 0437 | 0510 | 1432 | 1505 |
| 23 | 0309 | 0338 | 1552 | 1621 | 23 | 0340 | 0409 | 1505 | 1534 | 23 | 0413 | 0445 | 1433 | 1505 | 23 | 0438 | 0511 | 1433 | 1506 |
| 24 | 0310 | 0339 | 1551 | 1620 | 24 | 0341 | 0410 | 1503 | 1533 | 24 | 0414 | 0446 | 1433 | 1504 | 24 | 0438 | 0511 | 1434 | 1506 |
| 25 | 0311 | 0340 | 1549 | 1618 | 25 | 0342 | 0411 | 1502 | 1532 | 25 | 0415 | 0447 | 1432 | 1504 | 25 | 0439 | 0511 | 1434 | 1507 |
| 26 | 0312 | 0341 | 1547 | 1616 | 26 | 0343 | 0413 | 1501 | 1530 | 26 | 0416 | 0448 | 1432 | 1503 | 26 | 0439 | 0512 | 1435 | 1508 |
| 27 | 0313 | 0342 | 1546 | 1615 | 27 | 0344 | 0414 | 1459 | 1529 | 27 | 0417 | 0449 | 1431 | 1503 | 27 | 0439 | 0512 | 1435 | 1508 |
| 28 | 0314 | 0343 | 1544 | 1613 | 28 | 0345 | 0415 | 1458 | 1528 | 28 | 0419 | 0450 | 1431 | 1503 | 28 | 0440 | 0512 | 1436 | 1509 |
| 29 | 0315 | 0344 | 1542 | 1611 | 29 | 0346 | 0416 | 1457 | 1527 | 29 | 0419 | 0452 | 1430 | 1502 | 29 | 0440 | 0513 | 1437 | 1510 |
| 30 | 0316 | 0345 | 1541 | 1610 | 30 | 0347 | 0417 | 1455 | 1525 | 30 | 0420 | 0453 | 1430 | 1502 | 30 | 0440 | 0513 | 1438 | 1510 |
|  |  |  |  |  | 31 | 0348 | 0418 | 1454 | 1524 |  |  |  |  |  | 31 | 0441 | 0513 | 1438 | 1511 |

### Aerodrome visuals for familiarization

#### Distant aerial view

#### Close aerial view

#### View toward RWY 06

#### View toward RWY 24

## CHARTS RELATED TO AN AERODROME

Aerodrome Chart — ICAO		AD 2.ZZZZ-ADC

Aircraft Parking/Docking Chart — ICAO		AD 2.ZZZZ-PRKG-1

Aerodrome Obstacle Chart — ICAO Type A	RWY 06	AD 2.ZZZZ-AOC-1

Aerodrome Obstacle Chart Type A	RWY 24	AD 2.ZZZZ-AOC-2

Aerodrome Obstacle Chart Type B	RWY 06 SIDE	AD 2.ZZZZ-AOC-3

Aerodrome Obstacle Chart Type B	RWY 24 SIDE	AD 2.ZZZZ-AOC-4

Aerodrome Obstacle Chart Type B	CENTRED – SMALL SCALE	AD 2.ZZZZ-AOC-5

Aerodrome Obstacle Chart Type B	CENTRED	AD 2.ZZZZ-AOC-6

Visual Approach Chart — ICAO		AD 2.ZZZZ-VAC-1

## VISUAL SEGMENT SURFACE (VSS) PENETRATION

NIL